Topic. What is the brilliance of the beam?

Explanation. There is no one Explanation to this Topic. It depends on source type, wavelength and other factors like divergence slit settings.

Topic. How can one improve a poor R value of a crystal?

Does a poor R value have a serious implication on the credibility of the crystal structure?

Explanation. Yes, a poor R value indicates the model is a poor fit to the data, you will likely struggle with publication if it is very high. There may be many reasons for this so it is hard to say how to improve it, a couple of tips, check the atoms are the correct type and none are missing, check the data was measured properly, don't takeaways trust the expected chemical formula, sometimes things can happen in the reaction you don't expect.

Topic. What is the actual accuracy of determining the unit cell parameters, the CrysAlisPro program gives results of 0.0001A, but when measuring the same crystal several times or taking different sizes of crystallites from one synthesis, the discrepancies are much greater at the level of 0.01-0.001 A

Explanation. It is generally known that modern diffractometers tend to produce very low ESDS on cell parameters. Differences between crystallites may be giving you some insight into the spread in Topicuality of the crystals.

Topic. I have a crystal and its diffraction pattern is very good, but we can't get unit cells. What is the problem?

Explanation. There could be many reasons for this. Most likely candidates are twinning, or incommensurate behaviour

Topic. How the distance of the detector from the sample affect for the data collection?

Explanation. Several things, longer data collections, more separated peaks, lower background if there is significant signal from things like fluorescence, air scatter. There may be other factors.

Topic. I work on metal-organic frameworks and sometimes I get better Topicuality and size of crystals in 24 hours as compared to 72 hours. How can we explain that if crystal growth is directly dependent upon time?

Explanation. Metal organic framework materials are uniTopicue.  These are forming more of a polymeric crystal and so the standard things do not usually hold since you are trying to crystallize a material more all at once, not one molecule at a time.

Topic. There are times when I am doing solvent layering, for example: dichloromethane and n-hexane, upon slowly adding the n-hexane, I get cloudy solution which will disappear after some time, is this common?

Explanation. Yes, this is a good sign.  Stop layering, do not layer more hexane after it clears.   This is because the top layer of the DCM has more concentration of hexane and it is trying to ppt out.

Topic. There are times when I am doing solvent layering, for example: dichloromethane and n-hexane, upon slowly adding the n-hexane, I get cloudy solution which will disappear after some time, is this common?

Explanation. Thank you for the information and yes, I do get good crystals it I get the cloudy layer but I was thinking if this is common and why do I get the cloudy layer.

Topic. Do these methods of crystal growth also work for needle crystals or only plate crystals?

Explanation. Plate and needles differ in that it is which direction of the crystal grows faster.  Sometime change in solvent can effect crystal shape.

Topic. Any time frame to allow the crystal growth?

Does it depend on the kind of solvent used, concentration, or any other factor?

Explanation. Yes, to all.  all of these factors affect the crystal growth of the material.

Topic. Is growth of crystal depending on the outside environment? I mean outside temperature.

Explanation. Definitely, where you pace the vial, is there vibration, warm, cold, light.

Topic. I do get good crystals it I get the cloudy layer but I was thinking if this is common and why do I get the cloudy layer.

Explanation. Cloudy layer is because, one your concentration might be too high or compound not very soluble in lower layer.  Place a layer of Toluene between them and this will prevent the mixing at the surface that causes the cloudy precipitation to form.

Topic. Is there a duration reTopicuired for crystal growth prior to SCX diffraction analysis or just depend on virtualization of the crystals in solution?

Explanation. So the Topic is when are the crystals suitable for taking to put on the diffractometer.

1) Size, large enough to mount

2) Looks reasonable.

Topic. Is your approach different if you grow crystals from a solution and from a suspension with already existing crystals?

Explanation. If you are going to have crystals present, then most of the time you need the concentration such that not all the crystals will dissolve up.  There are two thoughts here, one is that the crystal provides a surface to grow larger crystals or two the presence of these crystals create the rest of the material to want to grow similar to these crystals.?

Topic. Is your approach different if you grow crystals from a solution and from a suspension with already existing crystals?

Explanation. The second one would be my case, powder Crystal A + powder Crystal B + Water = The crystals I want.

But not both A and B dissolve in water.

Topic. Apart from taking the crystals sooner, are there other ways to get smaller crystals (from the crystallization point of view) without crushing?

Explanation. The best method I find for collecting crystals, is to use siliconize vials where the compound does not stick to the surface of the glass, and generally do not use evaporation.

Topic. Of the typical crystallization methods, which would you recommend for the crystallization of mildly unstable cationic species? The Topicuickest method?

Explanation. Low temperature, saturate the solution and then place between -20 and-80 C.  Many crystals here at MSU have been done in pentane at -40 C.

Topic. Any suggestion for "magic solvent" for compounds insoluble in any solvents? In my case it is often carbon disulphide (CS_2)

Explanation. This is a difficult situation.  DMSO and DMF can possibly be used, but only small amounts of these solvents.  Good luck.

Nonpolar. Then DMSO/DMF does not help. Hex/Hept as well

Topic. Do you have any tips for layering techniTopicue when using really polar solvents, such as water?

Explanation. Water is a difficult solvent to work with to get crystals from.  (soluble hot insoluble cold is most used) layering with alcohols may work, but usually very long alkyl chain alcohols are needed to get the compound out of the water.  Or use Macro type method.

Topic. We are preparing healthy fat from vegetable oils. Is it possible to form crystals in fat?

Explanation. Your situation is very uniTopicue and also means that you need to purify as you crystallize.  I would say that low temperature is something you would want to attempt first.

Topic. How can I siliconize a vial?

Explanation. This is a standard method used in protein crystallization.  Check older papers to find a method.

Topic. Is there a "universal" seed we can use to help crystallise a material?

Explanation. No I have had many people tell me of thing they added to the vial to get crystals, but nothing is standard at all.

Topic. Determining the crystallinity of metal organic framework (MOFs) by SCX is often a big challenge often ends at PXRD analysis due to convoluted nature of MOFs, is there a way that your team were able to improve the determination of MOFs structure?

Explanation. We crystallize these from the reaction mixture, by slow cooling of the solution.  Often a powder diffraction is all you can get, this is a tough compound to crystallize.

Topic. Determining the crystallinity of metal organic framework (MOFs) by SCX is often a big challenge often ends at PXRD analysis due to convoluted nature of MOFs, is there a way that any team globally were able to improve the determination of MOFs structure?

Explanation. Exactly. We tried several time to get the SC but still working with different modulators to enhance the crystallinity.

Topic. Do you have any suggestions for improving crystallinity of a powder for microED, as opposed to getting single crystals for XRD?

Explanation. This is a very interesting Topic.  As microED becomes more popular, this will be something people will be after.  I find doing the same type of things here, but of course since you are not worried about size you can afford to have it crystallize Topicuickly.  Stir it in hot solvent?

Topic. What are the methods that we can use to convert amorphous materials to crystalline materials in solid state chemistry?

Explanation. Many people use annealing, Heating the compound up then letting it re-cool.  Some compounds need to get to melting point some do not.  This is a definite challenge.   Most I know about change the way the reaction occurs to try and generate non-amorphous material.

Topic. Is there any way to prevent twinning during crystallization?

Explanation. Yes, and no.  Often twining is an effect the crystal growth process and sometimes you cannot change the process, but a change of solvent often can help.

Topic. How we can crystallize a macrocyclic host with organic guest, like pillararene calixarene?

Explanation. Here is a paper in preferential crystallization of a calixarene and guest from a mixture.

Topic. Some polymeric compound is not dissolved any solvent that time what i used?

Explanation. it likely depends on the polymer length and type but not sure I can help here with this information.

Topic. The solvent system is part of the reactant?

Explanation. Not sure what this Topic means

Topic. Is there any way to crystallize ionic compounds (ionic complexes) while ensuring there are no lattice solvents?

Explanation. It may be difficult to 'ensure' this, try different solvent systems of varying polarity. You probably don't want to use something that will stick strongly, i.e. highly polar solvents. You may want to explore alternative approaches like microwaves and high pressure presses if you can't find a solvent system.

Topic. Can you talk a little bit more about vial siliconisation?

Explanation. Not sure what to add. Essentially, water contact angle to siliconized glass is higher giving a 'standing drop' which helps with vapour diffusion. It also prevents crystals from sticking to the glass.

Topic. Which could be the best strategy to measure organic crystals with synchrotron radiation, with wavelength generally speaking between 0.4 - 0.1 Angstrom?

Explanation. Those wavelengths are fine because beams at synchrotron are very strong. So short wavelengths are fine anyway.

Topic. you mean in terms of brilliance?

Explanation. Yes

Topic. Is there a way to measure the temp of the crystal directly instead of just the gas blown onto it?

Explanation. I'm not aware of it. The cryo-systems on the market all measure the temperature of the gas coming out. Now ... the distance to the crystal is so short, the speed of the cold stream high and the crystal so small that it is assumed the temp of the crystal is really close to that of the gas blown out.

Not really, low temperature systems are very good.  I place a thermal couple in the diffractometer and centre it and it reads a temperature within 0.5 degrees of the expected temperature, usually less.  I have done this with the oxford 800 and the 700.  The 600 the variation was almost 1 degree at times.

Topic. Why should higher temperature get rid of in commensuration and twinning?

Explanation. My best guess is that unit cell rearrange themselves as the temp increases. So they may yield to a together and more regular packing at higher temperature. Several crystallographers have told me that experienced such events ... sometimes. Worth a try.

I'm not an expert, but I was thinking about the electron density ellipsoids which are elongated at higher temperature, therefore, you'll have more overlap between the adjacent electron densities make them overlapped more leading to a single electron density (but not spherical) on your detector!

Topic. How can the users better customize the order of the data collection runs? The CrysAlisPro program currently seems only allow users to sort out the runs based on the theta angles or keppa angles. But can users sort out the order of the data collection runs with more freedom in CrysAlisPro program? For example, in APEX program, the users can sort out the order of the data collection runs with any orders that they want to use.

Explanation. The order of the runs is already optimized for (1) reaching completeness as soon as possible and (2) getting there as fast as possible. For instance, since omega can collect in both direction, a run might be situated to start where the previous run finished and collect this run counter-clockwise. As a result, I never change the order of the runs. But ... depends what you want to do. Why do you want to change the order of the runs exactly?

Topic. For studying temperature-dependent phase transitions it's mandatory to use the same one crystal for HT phase and LT phase? - or it's irrelevant and a number of different crystals can be measured.

Explanation. I'd use the same crystal. The thing is that some transitions may take place if the crystal goes through a certain series of different temperature. I suppose you could use different crystals but make the new crystal go through the same temperature set points and for the same amounts of time. There is also the issue of hysteresis. So again, make sure all crystals go through the same series of temp set points and for the same amounts of time.

Topic. Smaller crystals usually have longer measurement time. Is it not detrimental for the sample to expose it to X-ray radiation for longer duration?

Explanation. It may be, yes. But usually with small molecule crystals, radiation damage should not be your primary concern. If it does happen, then go ahead and use a larger crystal. Also, as I say in the presentation, I'm saying to keep experiment time reasonable, like from half hour to 2-3 hours max. DO not use a crystal that will reTopicuire to increase experiment time from half an hour to 10 hours. Just use common sense.

Topic. Smaller crystals usually have longer measurement time. Is it not detrimental for the sample to expose it to X-ray radiation for longer duration?

I work with MOFs and in my observation, smaller crystals exhibited less redundancy with the default mode. However, on choosing complete coverage mode, the time varies by a factor of 4-10 times, changing it from 1 hour with 20-40% redundancy to 6-7 hour measurement with 90-100% redundancy.

Any suggestions on that?

Explanation. Your low redundancy can be due to 2 reasons: (1) the symmetry of the lattice found upon indexing was higher than the true symmetry of your crystal. So the strategy was calculated with the wrong symmetry and the completeness was low at 40% or so. That would be why using 'complete coverage) improved your completeness. Or (2) your exposure time was too short and most of the high resolution reflections could not be collected.

Topic. How to figure out my crystal is strongly absorbing?

Explanation. The chemical content of the compound. Does it contain heavy elements? What are they and how many per molecule?

For an absorbing compound the ideal thickness is that which gives a transmission of 50% along the longest crystal direction.

Topic. Paratone N has been mentioned in the talk, it is Topicuite a nice mounting liTopicuid, though I think Hampton Research has discontinued the supply of Paratone N for some time already.  Just wonder anywhere else is still available?

Explanation. You are right, Hampton suggest this product as an alternative.

I have no personal experience of it, but have used Fomblin oil before. I think any inert mineral oil of sufficiently high molecular weight should work.

A colleague also suggests that STP oil additive and mineral oil (Nujol) are also options.

I have tried the little bit pricy cryo oil but may not be as good as Paratone N for many samples

Topic. Where can I find the information about the size of x-ray beam for my instrument? I have a Rigaku XtaLAB mini II.

Explanation. The beam is 0.5 mm and that beam is flat, unlike the beam of the Synergy S used in today's presentation. So ... 0.5 mm or 500 microns.

Topic. What are these horizontal and vertical shadows on blue screen?

Explanation. The horizontal shadow is the little gap between the 2 modules of the detector. All detectors are a mosaic of module stuck together. It appears wide on the screen because the image is shown in a large scale. It is smaller on the detector.

The vertical shadow comes from the beamstop.

Topic. For mosicity, there are three values: e1, e2, and e3. Should we look for smaller values of these three?

Explanation. Yes, usually these values are between ~0.5 and 1 degree. Beyond 1 degree, it is Topicuite a mosaic crystal.

Topic. What is the shadow on the diffraction images (not the beam-stop)?

Explanation. The horizontal shadow is the little gap between the 2 modules of the detector. All detectors are a mosaic of module stuck together. It appears wide on the screen because the image is shown in a large scale. It is smaller on the detector.

Topic. How to use de-icing function? And when should we use it?

Explanation. If your cryo-system works fine, you shouldn't have to use it. If it is not running so well, I'd use de-icing for longer data collection of at least 3 to 5 hours. But that depends on how bad your cryo-system is. I think the feature is in the Options dialog in CAP.

Topic. For some crystals, there seems no reflections at high theta area even very long exposure time is used. What should we do to such kind of samples?

Explanation. (1) Try large crystal. (2) Go back to the bench and try to find crystallization conditions that will yield better crystal. (3) You can also try a synchrotron beamline where the beam's flux is really high.

Topic. Could I know how to determine an optimal sample-to-detector distance?

Explanation. It is done visually: you check whether neighbouring reflections visually overlap or not i.e., they are a little on top of each other. But with Hybrid Photon Counting detector, you can collect almost ALL small molecule data at 30-35 mm.

Topic. Could I know how to determine an optimal sample-to-detector distance?

Explanation. Then use the closest distance possible ... unless you have a very long unit cell dimensions such as those occurring in protein crystals.

Topic. Regarding choosing the proper radiation source, don’t you think that the presence of heavy metal nodes within the MOF lattice could affect the diffraction when Cu radiation is used? Just curious.

Explanation. The ratio between the metal sites and all the other light atoms is usually low. Therefore, one can often use Cu for MOF crystals.

Topic. I observed sometimes that many peaks fitted to my crystals but couldn’t find any space group. What is your opinion about this and what we should do with them?

Explanation. Impossible to say without seeing the data. Could be a bad crystal, multiple diffraction patterns that confuse the indexing algorithm, or even a badly calibrated diffractometer where the detector distance or direct beam position is wrong.

Topic. In which case is it better collet full data? And which case is better long redundancy data?

Explanation. But you want a higher redundancy when (1) You need to get the absolute configuration of a compound, (2) you are dealing with an X-ray absorbing crystal. A higher redundancy will lead to a better empirical absorption correction which is done on symmetry-related reflections. And maybe (3) if you have a weak crystal that yield weak data at high-resolution, increasing the redundancy will improve a little the I/Sigma at high resolution.

Topic. Atlas's detector strategy says the image binning 2x2 4x4, what does that mean?

Explanation. It puts together a grid of 2 pixels by 2 pixels i.e., making a larger pixel. This is the default binning for many compounds. You switch to 1x1 binning i.e., using each individual pixel as they are, when reflections are close together and you need to resolve them better. So typically, 1x1 binning is used for unit cell with large dimensions.

Topic. Why large crystals, say 200um, are inappropriate for X-ray diffraction experiments, if the shape of our crystal is regular, e.g. a cube, you can introduce an absorption correction in CrysAlisPro. What is the main substrate for which it is recommended to use crystals with dimensions of about 50um?

Explanation. The smaller the crystal, the sharper the reflection.

Topic. In the theory of static diffraction, the more planes involved in scattering, the narrower the diffraction peaks, so why is this so? Also the high-resolution reflections are weak, isn't this a disadvantage of using small crystals?

Explanation. As crystals grow, unit cells will statistically adopt sub-optimum orientations, increasing mosaicity and thus decreasing reflections' sharpness. It is not always like that indeed, but usually. Also the longer the crystallization goes on, the more likely defects will statistically occur, again worsening the shape of reflections. As for experiment time, you are correct. Therefore, you must increase the exposure time. That's why I say in the presentation to use smaller crystals if diffraction from that sample is fairly good. You can't use small crystals if they already diffract weakly.

Topic. The detector distance from the sample is related with the nature of the crystal or with the eTopicuipment setting?

Explanation. Both. It depends (1) on the type of the detector and (2) on the unit cell dimensions of the crystal analysed.

Topic. Is there a reason you would ever change the detector distance?

Explanation. Overlapping reflections coming from a long unit cell dimension (like a protein crystal) or a twin. If none of that, 30-35 mm for a Hybrid Photon Counting detector will cover the vast majority of your samples this is a different story with CCD and CMOS detectors that must be put a bit further back

Topic. Is there any situation in which we have to use smaller or bigger scan width?

Explanation. Not with modern diffractometers.

Topic. What is acceptable I/sigma in terms of publication, 15?

Explanation. 15 is good, but you may not always be able to get there if you have a weak crystal. Try to get at least 10.

Topic. Would you suggest to always use the theta-dependency on the calculation of exposures on the collection strategy or just for low diffracting crystals?

Explanation. Always use it.

Topic. What is the difference of calculating for a 1 Laue group or for a "sphere"?

Explanation. Laue group 1 is really P-1. A sphere is P1.

Topic. Why full sphere is more than double triclinic? Shouldn't it just be two times?

Explanation. It should be about twice more. It depends how the strategy was calculated. Different 'seeds' can be used to calculate the strategy for a given crystal.

Topic. What is your closet detector distances for all HyPix type detectors (curve and flat ones)?

Explanation. For flat HyPix, as close as possible to the beamstop. Usually 31-33 mm.

For curved detectors, strategies are better with a distance of about 40 mm.

Topic. How to deal with very unstable crystals, which decay before completeness ~98% is achieved? is this possible to collect a few datasets (with completeness ~ 80-90%) from different crystals of the very same compound, the merge these datasets and to solve and refine the structure using this merged dataset?

Explanation. The CrysAlisPro software allows to merge different datasets.

Topic. During strategy collection it shows strong diffraction but very low I/Sigma. How will I increase the I/Sigma? The longer data collection could resolve the problem or something else. Please suggest.

Explanation. Yes, longer exposure times.

Topic. I have crystal which have very less size. it is a sheet type crystal but thin. Not coming in the loop. How to collect data?

Explanation. You can also try using MiteGen micromesh loops.

Topic. It seems that is not necessary the beamstop, is that correct?

Explanation. That’s correct. As there is no air in the diffractometer which could cause background scattering, and the detector is radiation hard and does not have bleeding effects, there is no advantage in using a beamstop.

Topic. How to load the sensitive samples?

Explanation. If you mean samples which are vacuum sensitive, you would use cryo-transfer. If your sample is air- or moisture-sensitive, you could prepare the grids in a glovebox, insert them in a small TEM grid storage box and proceed with cryo-transfer procedure like for vacuum sensitive samples.

Topic. Given access to well-crystallized inorganic samples down to millimetres in size, what are the advantages of electron diffraction over X-ray diffraction? If it allows us to determine the absolute structure with better accuracy, how significant is this improvement?

Explanation. Generally, for large crystals, electrons are not suitable due to their strong interaction. 3D ED is rather pertinent to samples which would otherwise not be amenable to single-crystal diffraction at all. Electrons can still yield some additional insight, such as charge states (due to scattering from the Coulomb potential) and a strong indication of absolute structure. However, for crystal sizes exceeding several microns, you’d rather want X-rays.

Topic. What is the significant difference between TEM and this ED?

Explanation. In terms of hardware, while the overall electron-optics and gun is similar to a TEM, the configuration is very much optimized for electron diffraction. Also, the type of detector is totally different from a TEM camera. Finally, the control system and user interaction with the hardware is very different and resembles more closely a SCXRD machine than a TEM.

Topic. What is the purpose of doing GRID and what additional information do we get besides doing SCXD?

Explanation. Different than in SCXRD, we do not just mount one single crystal due to the size, but thousands of crystals on a TEM grid. The grid is our sample holder. SCXRD has the 1-micron barrier. Samples smaller can't be measured with X-ray, hence ED is a nice new tool to get the crystal structure.

Topic. What is the difference between the HRTEM and ED diffraction? Is it possible to collect the ED diffraction in HRTEM?

Explanation. If you’re referring to the instrument here: ED data can be taken on high-end HRTEM machines, but it takes much more skill and effort, as those machines are not optimized for it.

Topic. As an emerging techniTopicue, what are the main limitations of 3D ED techniTopicue compared to traditional SCXRD?

Explanation. Certainly, some limitations are about the sample environment, e.g. the vacuum in the diffractometer - which can be worked around by cryo transfer. Another limitation is the unavailability of multi-axis goniometers, again due to spatial constraints and the reTopicuirement of vacuum transfer. This may necessitate merging of data from different grains, which is however straightforward.

Topic. What is the beam size and the resolution of the ED?

Explanation. On the Synergy-ED, the minimum beam size is 20 nanometres. However, we mostly use the selection-aperture approach, where the diffracting area goes as low as one micron, matching typical crystal sizes and the gonio’s sphere of confusion.

About resolution: if you refer to the diffraction resolution, this can be arbitrarily set by varying the detector distance.

Topic. How is the integration process? Can the structure be solved in the traditional way without using Autochem?

Explanation. Yes, absolutely. Virtually all standard crystallographic software will work with ED data.

Topic. How long/difficult is cryo loading a sample in comparison to room temp?

Explanation. The sample preparation is more complex and it also takes longer for temperature stabilization. However, after doing the sample preparation a few times, it does not take much longer than with the ambient temperature specimen holder. In a normal working day, one can do 3-5 cryo samples. Standard RT measurement, maybe 6-8 samples.

Topic. Do you have any example to be made by FIB machine without using grid?

Explanation. Even for FIB-milled samples, you would first deposit them on a grid in a typical workflow. We do have examples for it. See e.g. in Truong et al. doi:10.3390/sym15081555

Topic. ED resolution is determined by the distance like XRD. Unlike TEM determines the resolution by applied voltage?

Explanation. I’d say that it is generally hard to compare the meaning of “resolution“in ED and EM; in both cases, the achievable resolution is affected by a lot of factors.

Topic. What is the advantage of ED compared to SCXRD?

Explanation. In addition, bulk analysis. If you have a sample which has different phases, most likely you could catch it during screening and later measure whole datasets of the different polymorphs or phases.

Topic. What are "acceptable" R_int values for ED? Especially in the context of merged datasets?

Explanation. There are no “hard“ and generally accepted standards here. For datasets strongly affected by dynamical effects, values as high as 0.5 are still not indicative of bad data. That said, for typical organic samples, anything higher than 0.3 makes us suspicious.

Topic. You just said that the R value usually decreased with increasing redundancy and that merging several grains would thus lead to a slightly lower R. Why is that the case? How are redundancy and R value related to each other?

Explanation. R_int by definition will increase with higher redundancy, even if observations are perfectly consistent save for their noise. Hence, we recommend to rather look at R_rim, which adds a normalization factor that removes this bias (but not systematic increase due to dynamical effects)

Topic. Is the wavelength of the ED changeable?

Explanation. Yes, by varying the acceleration voltage.

Topic. What does the gonometric and stereographic view mean/how are they helpful in Ewald Explorer?

Explanation. Both are kind of legacy views and more common in mineralogy.

The gnomic projection can be used as alternative method for lattice indexation. Particularly for bad data, this is a great tool. You can look for "triangles" in the diffraction spots and index the cell with gnomic vectors.

Topic. Are peaks that do not meet the Laue conditions for the determined crystal lattice also registered in the "peak list"?

Explanation. Yes. All peaks that meet certain criteria are extracted from the frames.

Topic. When we apply intensity and d value filters?

Explanation. Intensity: for filtering weak features to find a lattice in the stronger reflections or filter strong reflections to find a weak super lattice.

D-value: for filtering out powder features like ice rings.

Topic. Is there a way to remove ice or powder rings from your data besides just excluding all the frames where they're present?

Explanation. use the skip d-value tool in CrysAlisPro:

right-click the frame, activate line-select, select the ice ring, right-click and skip d-value

Topic. I was wondering if modulated structure can be fully analysed with CrysalisPro/Olex2. You showed that modulated lattices (Topic-vectors I presume) and their satellite peaks can be indexed and processed but I was wondering if is possible to fully describe modulation function(s).

Explanation. Modulated structures can be processed in CrysAlisPro in combination with Jana (2006 or 2020)

Topic. After integration, the main screen shows only the results peaks.

Explanation. Yes

Topic. Can we use the Bruker apex data to integrate in Rigaku CrysAlis pro? and how?

Explanation. You can. There is a frame import tool on the left hand side icons of the main window which guides you through the process. Note that the instrument model may need to be refined or preloaded from a known sample like the standard crystal (ylid) to achieve good results.

Topic. Is there than any specific correction tips for needle like crystals or plates? To compensate for the shorter / longer direction?

Explanation. I would recommend to cut the crystals to a length suitable for your setup. If this is not possible, you could try centering your needle in a way that you only measure it at the very tip.

Reduction of absorption can further be achieved by choosing a shorter wavelength, and higher redundancy in the dataset will help with empirical scaling.

Topic. Won’t we lose information about any defects/anomalies or impurities if we choose the outlier rejection?

Explanation. Typically, you measure redundant data that means rejection of one reflection will not impact data Topicuality due to symmetry reasons.

Topic. Can the corrections that are used for X-ray data directly be used also for electrons?

Explanation. The empirical corrections employed for ED are practically the same as for X-ray data. Numerical absorption correction is unfortunately not possible.

Topic. How we can filter the R_int?

Explanation. One chooses the filter and a rejection criteria for it. R_int is just one of those and should be used with caution.

Topic. In what cases is it better to use Analytical absorption (Clark & Reid) and in which Gaussian grid (Numerical integration) correction?

Explanation. Gaussian Grid allows for more grid points and beam profile correction, so that should be chosen by standard.

Topic. If you have a crystal that has 2 possible space groups like C2/c and I2/a which one would you choose? And why?

Explanation. The comment rule is to choose the space group setting with the smaller beta-angle.

Topic. I am working on MOFs and the PXRD results shows sharp peaks which is an indication of crystallinity within the MOFs matrix. However, to get the SCXRD using XtaLAB Synergy in my school is what we are yet to achieve. The problem is that; It is not showing any diffraction after mounting the sample. Do you think using the concept of R_int filter and other correction parameters could improve the crystal Topicuality?

Explanation. Tim, if you have no diffraction on the images, there's nothing the software can do. A few things:

1- Is it possible the crystals decay when you take them out of their mother liTopicuor to mount them on a loop?

2- Did you do something with the sample after you measured PXRD data? If the crystals don't show diffraction data, then they are no longer crystalline.

3- Is it possible that you have other species in your sample which are crystalline and they are the ones showing powder peaks? Remember that with PXRD, you measure data on a bulk sample, not just 1 crystal. What are the unit dimensions from the PXRD data? Are they consistent with a unit cell for a MOF sample?

4- Try longer exposure times or larger crystals.

Try Cu radiation if you are using Mo now.

Topic. If sometimes the software changes from your initial unit cell to the Niggli cell between the Niggli and lattice screen, is there a way to stop this/a reason the software does this?

Explanation. You can restrict CAP by choosing a "user cell" during unit cell indexation.

Topic. How do I decide if it's a centrosymmetric or non-centrosymmetric on a crystal?

Explanation. The "E^2-1" value will indicate this. A further indication is the automatic structure solution from AutoChem and of course the data reduction tables and merging statistics. Crystal structure refinement will tell you again about being centrosymmetric or non-centrosymmetric.

Topic. Which files in the data folder that record the data measurement detail, data integration and correction details?

Explanation. The "Data Collection" tab in CrysAlisPro contains all necessary information, also you have access to the used run-list.

Also, for a lot of detail on the processing, the <sample_name>_red.sum file (it is a text file) can be studied. This file is also displayed by the "Inspect data reduction..." button.

Topic. Does the face indexing always improve the R_int in the end?

Explanation. Yes, the more you scale the dataset, the lower is R_int. You are not looking for the lowest R_int though when scaling your data!

Topic. In Bruker software system, there is .ls file which records all details of the data integration and  .abs file which record all details of absorption correction. Are there similar files created by CrysAlisPro?

Explanation. The "Data Collection" tab in CrysAlisPro contains all necessary information, also you have access to the used run-list.

Also, for a lot of detail on the processing, the <sample_name>_red.sum file (it is a text file) can be studied. This file is also displayed by the "Inspect data reduction..." button.

Topic. How accurate does the chemical formula need to be for the absorption correction? What about if we don't know the exact composition?

Explanation. If you choose the wrong absorption coefficient during scaling, you will generate an alert in CheckCIF. The easiest would be importing the formula from your refined crystal structure (.res file).

Topic. How is it that we lose the phase information during data collection, leaving us with the ability to measure only intensity and not phase?

Explanation. We can only measure the intensity of light as it hits a detector. We know that there *is* light, but we can't measure at which 'state of the wave' the light wave hits that detector. Light waves are just too fast!

Topic. If in XRD or PXRD, we can determine our refinement is good through the chi sTopicuare value between 1.0 to 1.5. How can we determine that our refinement results on this single crystal are good and valid? Are the parameters the same or do they have different parameters than XRD refinement in general?

Explanation. I am not sure what the Topic is. There are many different indicators that tell us whether a structure is valid or not. Arguably, the most important one is the R1 factor, but wR2, GooF etc. also tell us something. I think the most important indicator is the residual difference map -- Ctrl+M

Topic. I have come across some cases where "add H" option dose not add hydrogen. Can be done in just situations?

Explanation. You can always add hydrogen atoms geometrically if you know the relevant 'hadd' command: 'hadd 137' will add three atoms to a selected -Me carbon, or 'hadd 23' will add two hydrogen atoms to -CH2-CH2- sp3 carbon bonds, 'hadd 43' will add 1 hydrogen to phenyl rings etc. There are some YouTube videos on this on Rigaku channel.

Topic. I have Topic about validation and goodness of refinement in single crystal with this powerful software. If in XRD or PXRD, we can determine our refinement is good through the chi sTopicuare value between 1.0 to 1.5. How can we determine that our refinement results on this single crystal are good and valid? Are the parameters the same or do they have different parameters than XRD refinement in general?

Explanation. There are many different indicators that tell us whether a structure is valid or not. Arguably, the most important one is the R1 factor, but wR2, GooF etc. also tell us something. I think the most important indicator is the residual difference map -- Ctrl+M

Topic. How do you deal with it when it makes impossible bonds that shouldn't be there?

Explanation. You can set the maximum number of bonds for any one particular atom (select it!) and then type 'CONN 4' -- this will then only show four bonds to that atom.

Or: "conn $Co 6" -- all Co atoms will have no more than six bonds. Or "conn $M 8" -- no metal atom will have more than 8 bonds etc.

Topic. In general, how many atoms (or may be the number of reflections) can be handled by olex2.solve (probably not as many as a small protein?)?  There is a limit for XS around 100,000 I can't remember the exact value and need to set the -b number somewhere for a very big structure, while getting a warning.

Explanation. Hello Herman! olex2.solve is a very simply charge flipping implementation -- probably you'd use ShelXT! However, all dual space solution programs will get Topicuite slow as the unit cell becomes big. ShelXS is probably still a good choice, or ShelXD, which (if I remember correctly) was made specifically for solving small proteins

Topic. What information can we get from the R merge vs resolution plot?

Explanation. How consistent eTopicuivalent reflections are at which resolution. At low resolution (measured near the centre), they should be highly consistent (low R_merge). At high resolution, they will typically become less consistent -- but the better you can measure weak reflections (i.e. deal with the background properly, have enough diffraction etc.) -- the better your R_merge will be.

Topic. I was wondering if is possible to group lines in more than one group (drawn for highlight two differently oriented 1D-tubes in the crystal packing). When I try to do so and I generate the TIF file all the lines will be drawn with the same colour.

Explanation. you can do any number of different lines styles by giving your lines names:

line -n=blue_line

you can then change the graphical properties of this line.

next time you want the same kind of line, simply use

line -n=blue_line

These lines should be remembered between Olex2 restarts, but can also be made part of a style.

Topic. Why does the R_int not appear in the example?

Explanation. Because the data have been merged after data processing -- something that should really never ever happen.

Topic. How do we know if there's solvent molecules in our crystal structure? And can I know how can I utilise the function of 'Use Mask'?

Explanation. Well... if there is something that isn't connected to your main molecule, and is a solvent, then that's a solvent molecule in your structure. Sometimes it's not easy to figure out *what* that molecule is, though == solvent molecules can be highly disordered. They may also be counter-ions, of course.

Topic. Why does the size of TOPIC peaks automatically become smaller during the course of refinement?

Explanation. They should not do that --but I've noticed this, too -- sometimes. It must have something to do with switching between 'pers' and 'anis' etc. I will need to investigate this behaviour. In short: it's a bug.

Topic. Can we impose atomic ion scattering factors for olex2? e.g., when solving the structure of NaCl, we want the program to take into account f_calc atomic scattering factors for Na+ and Cl-, respectively, is this possible?

Explanation. Yes, these are tabulated and available -- but I'd have to look up the syntax of how to include them.

Possibly a good way to really get to the bottom for these sorts of things, you could consider using non-spherical form factors (e.g. NoSpherA2) for the refinement.

Topic. Why there is red region around cadmium in this particular structure?

Explanation. The absorption correction hasn't been done properly in this dataset.

Topic. In the case of highly disordered atoms, sometimes when I try to add the Hydrogen atoms, Olex gives an error about the number of bonds. For example, with tetra butyl ammonium, one of the terminal atoms is very disordered. It is an sp3 with 3 H atoms, but a cannot add the H atoms in both disordered carbons

Explanation. The most common reason for this is that the disordered bits haven't been put into the correct PART. Often (and I suspect that's what happened in your case), the whole thing needs to be in PART -1.

Topic. Hi Prof Puschmann, why Olex2 uses 3sigma rather than 2sigma of I/Isigma(I) to determine if the signal detected is basically noise? Can we change that in Olex2 to let the program to use 2sigma?

Explanation. If you mean the line in the display of the plots: that is just a line; it doesn't actually do anything. 2 sigma *is* used for calculations, for example:  R1 was 0.0258 (I≥2 sigma(I))

Topic. What is a good R1?

Explanation. That very much depends on the system, but the colours in Olex2 are a good guide for 'standard structures'. Green is good, orange not so much and red really needs to be justified. For example, in a very large cell, R factors can be higher than one would expect.

Topic. How to correct "warning: 0 atom may be split and 1 atoms NPD" error during refinement?

Explanation. You should never really get N.P.D atoms. If you do, you may have a wrong atom type there. Twinning could also be a reason for N.P.D atoms.

Topic. How do you explain the negative potential around the Cd?

Explanation. The absorption correction was probably not done well in this dataset.

Topic. XT is very powerful though I found sometimes it failed to solve high symmetry structure while XS gives the Explanation straightly (so just try all program available, in the old day I just use BLOC to deal with the very big structure and matrix -b can be adjust to include more reflections.  Dose olex2.refine has such limitation?

Explanation. No -- there is no such limitation in olex2.refine. However, in the current version, it can become too slow to be practicable. In the next update this will be fit, and olex2.refine will also be suitable for large structures (already out in the dev version)

Topic. Is Olex 2 able to handle Electron Diffraction data?

Explanation. Yes. It works well.

Topic. How can the water molecule be refined? The ellipsoid is large.

Explanation. It is free to 'move about' -- sometimes, refining it as disordered may help --otherwise restraints could be used. However: I'd just leave it as it is: the large ADP carries some information, after all!

Topic. What can a high value of the GooF parameter (above 1.2) mean with relatively low values of R1 and wR2 (below 4%), should we worry about it?

Explanation. This usually means that the weighting scheme has not been switched on.

If it *has* been switched on, and GooF is still showing as large, then there is something strange going on with the e.s.d. values of your measured reflections.

Topic. Is there a way to visualize the symmetry elements in olex2?

Explanation. No. We are not currently planning to implement this. However, if you want to know if an atom is on a special position, simply select it and type 'degen':

Mn1 [+X,+Y,+Z] 2

-X,1-Y,-Z

Site constraints: X=0, Y=0,Z=0

Or press CTRL+G, and you will see 'growing bonds' if it grows.

Topic. Does OLEX2 automatically give the site occupancies of the disordered atoms?

Explanation. Yes, but you can set them manually too: 'part -1 10.5'

Topic. That, R, Rwp and GooF give us that information. Based on your experience is there a certain value parameter for some of these indicators to be considered good?

Some journal literature that I read for R or Rwp is said to be good if the value is below 10% but this is for refinement on XRD Pattern, is it the same in this case?

On the other hand, for this GooF value, I have not found any literature related to what number of GooF indicators is said to be good.

Explanation. GooF should be 1. Depending on the size of the e.s.d of a reflection, it is either up- or down-weighted. I.e. if we know that a reflection has been measured badly, then we don't give it as much weight in the refinement as we give a reflection where we know it's been measured well.

Topic. Could you please explain the importance of selecting or deselecting EXTI?

Explanation. If the physical phenomenon of extinction is present in your experiment, then you generally want to refine the EXTI parameter. If this refines to 0, then, no, there is no point in it. But if it refines to a value around 3 times larger than the e.s.d, then you should reap it in the refinement. This will make a big difference for your R factors, but is especially noticeable in the residual density map.

Topic. What do you mean working with merged datasets should not be done? Do you mean merging reflections by symmetry or datasets?

Explanation. The data reduction should always return un-merge, un-cut reflection information.

Topic. Mercury from CCDC would be one of the software you can use to visualize the symmetry elements?

Explanation. Indeed.

Topic. Right now it always shows error warnings in Olex2 when you want to refine the structure as a twin using HKLF 5 file after you use Mask function. And sometimes the opposite wat also brings errors (refining the structure as a twin using HKLF 5 file and then use Mask function). Is Olex2 going to solve this problem in the future's upgrade?

Explanation. I think you *are* using an old version. You can also always use PLATON (after LIST 8) for twinned structures.

Topic. Is it possible to calculate the void using olex2?

Explanation. Yes, there are different ways of calculating the voids -- it's under Tools > Maps

Topic. How the max peak be taken care of?

In my structure I have seen only one counter anion. But it has to be two?  How can I proceed? Please suggest.

Explanation. 'refine 4 30' will refine four cycles and show 30 peaks.

Topic. Missing FCF Refl between Thmin & STh/L= 0.600             6 Report

Explanation. This means that there are reflections up to the resolution limit (Sin theta/lambda = 0.6) are missing. For example, it may be that you have a reflection under the beamstop that was not measurable. You are more likely to see this for lower symmetry but the Explanation to this is to extend the data collection strategy to measure the missing data.

Topic. Missing FCF Refl between Thmin & STh/L= 0.600             6 Report. How to fix this? (2) Large K Value in the Analysis of Variance ...... 4.131 Check.

Generally getting these two alerts in almost every structure?

Explanation. This means there is an unusual statistical spread in your data. There are numerous reasons this could happen. Potentially the data have not been measured with long enough exposure times. It is a little difficult to diagnose without the data from your system.

Topic. How to reduce R_int value?

Explanation. It is a bit of an open ended Topic. First we need to know why R_int might be high. The most likely cause is that you have data that does not have a good enough I/sigma. The shorter your exposure times, the more likely the I/sigma is too low and the intensities are less reliable. Try running longer experiments, in particular with longer exposure times.

Topic. Sometimes when I check the finalized cif file in checkCIF, it shows warnings "PLAT950_ALERT_5_G Calculated (ThMax) and CIF-Reported Hmax Differ", "PLAT955_ALERT_1_G Reported (CIF) and Actual (FCF) Lmax Differ by", and "PLAT992_ALERT_5_G Repd & Actual _reflns_number_gt Values Differ by". But during finalization process of cif file, I did not manually edit and change any related values of cif file. So can I know what is going on with my cif file? Why did I get these warnings?

Explanation. The most common reason here is that there are rounding errors or rather differences between your refinement program and Platon. This alert is marked as G which means you can check it but normally this is not an issue.

Topic. How can we remove the residual density alert, already we assigned the solvent but still this residual density alert is coming? After sTopicueezing the solvent also, it is coming?

Explanation. Again this would depend on the reasons behind the residual density being high. The most common reasons are missed atoms, incorrect atoms, and Fourier truncation peaks which are typically associated with heavily absorbing elements. Check your atoms are correct. In the last case they are usually symmetrically arranged on either side of the heavy element. Try checking your absorption correction. This error is far less likely when using the Analytical or Numerical methods for absorption correction as long as they are done properly.

Topic. I have a structure which has large number of water molecules. Even after adding hydrogens one by one to these water molecules, I get alerts b, a) Short Inter D-H..H-D, b) D-H Bond without Acceptor. In this case, is it necessary to fix these hydrogens using dfix, or is there any other way to do it, I have already tried had-r and had commands?

Explanation. You may need to build up your hydrogen bonding network more carefully -- it helps to check for symmetry eTopicuivalence atoms nearby.

Topic. Lately I have had this alert, "PLAT142_ALERT_4_G s.u. on b - Axis Small or Missing," it is Alert G, but I don't know how to prevent it from appearing.

Explanation. Is your space group correct? This could happen when an axis is eTopicuivalent by processing but later symmetry was changed to lower and there is no esd in the refinement files. I would expect this is unlikely to happen if only programs handle the files. Is the esd actually missing?

I also always see this alert. It seems the CrysAlisPro can yield unit cell parameters with very small esd. While as a user, I think this is a good thing, but it seems checkCIF does not "believe" the esd can be so small...

Actually this one is the other case where we have reflections beyond our resolution limit being missing. This is normal for area detector experiments since we cannot easily collect data only up to a given limit and not beyond it. Note that it is a G alert so you do not need to worry too much. Just check it is OK and acknowledge it. It would be upgraded to a C or higher alert if it is serious enough.

Topic. How can we reduce the I/Sigma? sometimes alert comes corresponding to it? how we can clear it?

Explanation. Reducing I/sigma is a bad idea. You want to increase it either by increasing exposure times or collecting more redundancy.

You need a longer exposure time. There is nothing you can do 'after the event' -- just re-collect new data, making sure to choose a suitably long exposure time. The pre-experiment will help with this.

Topic. While merging data collected from the different crystals (crystals of the same compound obviously) does R_int usually increase?

Explanation. Potentially. You may have crystals of different sizes which can have conseTopicuences for absorption correction, exposure times and so on. You therefore may end up with slightly different behaviour in each crystallite. There are no real yes no Explanations in this though.

Topic. The 0.84 Angstrom resolution threshold is valid for both CuKa and MoKa sources or just for CuKa?

Explanation. For copper, for Mo it is 0.75. For Mo 0.79 Angstrom can also be applied

The minimum reTopicuired resolution is independent of the radiation, and it's always 0.84 ANG. For Mo, this is 50 degrees in 2 Theta, and in Cu it is ~135 degrees in 2 Theta.

Topic. It seems checkCIF uses the rule of I/sigma > 2 rather than 3. So if high resolution srea data with I/sigma <3 were cut off by "OMIT", checkCIF will show a alert "PLAT909_ALERT_3_G Percentage of I>2sig(I) Data at Theta(Max) Still ", which warns some observed data, rather than the noise, was removed improperly. Should we take care of this warning?

Explanation. G alerts are the least serious. If you know what you are doing it is up to you how to handle this. My general advice would be to not throw away data unless you absolutely have to. Simply lowering your R1 from e.g. 6% to 5% is not really a good reason to do this on its own. Protein crystallographers will use data down to I/sig =1 for example.

Topic. PLAT230_ALERT_2_C Hirshfeld Test Diff for O00A --N00Z'' How can we remove this alerts?

Explanation. Try to restraints the thermal parameter using SIMU or RIGU.

It means that two atoms in a bonding pair vibrate differently, which is not physically sensible.

Check the atom types are correct. Check occupancies are correct. Check ADP behaviour is not unexpected. Then use restraints if everything checks out.

Topic. Apart from the known Acta Crystallographica Sections E and C, are there any other journals that you could suggest which focus mainly on synthesis and crystallography?

Explanation. It depends on what you are doing. Maybe CrystEngComm.

There are Topicuite a few journals.

Topic. PLAT430_ALERT_2_B Short Inter D...A Contact O007 ..O008Why we are getting this alerts and how can we remove?

Explanation. Probably due to misplaced H atoms.

This is saying that you have a short hydrogen bond, check for disorder, and check hydrogen atoms are in the right place. You may need to manually place hydrogens to achieve proper placement

Topic. How can we justify the high R_int (15-20%) in case of merged data? Is it enough to explain that it was not simply possible to collect full data from one crystal so the few data sets were merged? And that's what the high R_int is observed.

Explanation. Generally everything can be justified to a referee, whether they will accept depends on the scientific basis for your justification. Please note that in CrysAlisPro we have tools to merge data from multiple crystals and the individual crystallite Topicuality can be assessed with respect to the group. You may find some of your crystals have a poorer agreement or Topicuality than the group. It may be advantageous to remove crystals from the group that are a poor fit.

Topic. Is the high resolution structure gotten from high angle diffraction? and that is determined by the Topicuality of the crystal?

Explanation. High resolution means high theta. This is the axis around which the detector rotates. Higher resolution gives you smaller and smaller details in the data. E.g. Electron density reTopicuires very high resolution data. Your ability to measure high resolution may be affected by crystal Topicuality or temperature. Low temperature data collection usually helps us achieve higher resolution

Topic. I have collected a data using Mo source and it shows NPD. What should be done next? I have used low temperature also. yes,

Explanation. Hard to Explanation without more details. I can only offer some pointers. Are all of the atoms in the structure correct? Have you missed a very heavy atom? Is the crystal diffracting to high resolution?

Topic. all atoms in the structure are correct.

Explanation. From my experience, for many coordinated complexes with a metal centre (the heaviest atom in the structure) I sometimes suffered NPD for this heaviest atom while using 100K but it behaves well while using 173K instead, just FYI.

May be the scaling problem either.

Topic. What type of restrains and constrains are helpful to try and stop the hydrogens from rotating in your last example?

Explanation. You could try a restraint which uses a known hydrogen bond acceptor, if you cannot find a way to restrain it, use AFIX 33 instead of AFIX 137. AFIX 33 will not allow rotation and place in idealised positions.

Topic. For case study 5, why can't both Cl atoms are on the same C atom instead of 1 Cl and 1 F atoms?

Explanation. The model is describing disorder. It is not that there is one molecule with both Cl and F bonded to that carbon. We have two molecules superimposed on each other. One molecule has F on the left, and Cl on the right and the other has Cl on the left and F on the right. I hope that makes sense. Imagine that alternating unit cells have one of those two molecules and the next would have the other.

Topic. How voids and solvent accessible voids are different from each other?

Explanation. It is called a solvent accessible void because it is large enough to accommodate a solvent molecule. There will be small voids in all structures but they are small and even H2O would not fit in there.

Topic. So can we ignore voids?

Explanation. If you get an alert from Platon, you need to explain it. So no.

Topic. How the alert max peak 6.2 could be removed from check cif of an inorganic complex?

Explanation. Yes -- ALWAYS do a proper face absorption correction!

Topic. On one of the slides on resonant scattering, there is a flow chart at the bottom-right reading "anomalous dispersion, anomalous scattering, inelastic scattering, and resonant scattering". Are these all different phenomena? Do the arrows between the circles (going from left to right) have any meaning, in the sense of "leads to", "occurs prior to", "is larger than"?

Explanation. No, they are just different names used by different fields of science and depending on the text-book. The last two can have more general meaning if you don't talk about the x-rays only

Topic. What cause, or what should be done with, the alert: wR2 reported and calculated differ?

Explanation. Can you provide the alert number for this? Without it is hard to guess what can be a problem.

Have you checked CIF with or without HKL data? In the latter case, make sure that your CIF matches the submitted FCF file or that the FCF block in CIF has been updated.

In general, that means the value you put in CIF does not match the one that check CIF can calculate from submitted data.

Topic. From previous lecture of 3D-ED, data from a single grain will probably not guarantee high enough completeness. Is this going to negatively affect the determination of the absolute configuration?

Explanation. Yes, but it is not as bad as you could think. I will show some case studies later in the lecture. You can always replace the completeness with resolution. Data is data; you just need enough of it.

You can also load several data sets into Jana2020 to increase completeness, you just cannot merge them beforehand. Jana can then refine the structure on all the imported datasets simultaneously.

Completeness is especially important in the initial solution step. Specifically, for absolute structure, it is not that critical, as long as sufficiently many reflections recorded along zones with strong dynamical effects (i.e., those with many excited reflections) are included in the refinement.

Topic. So, in the Experiment Strategy, is 'Chiral only' then indicating use of Sohncke space groups only?

Explanation. Yes.  You can select from all non-centrosymmetric groups and Sohncke SG. The former will also allow polar space groups.

Topic. How can we change flack parameter?

Explanation. It will be printed out at the last cycle of refinement if you use shell or olex2refine. Most of the programs will include it automatically in the summary of the refinement of non-centrosymmetric groups. In Olex2 you will notice an extra box in the section where you have R_int R1 I/sig etc.

Topic What is cu source of radiation hope it’s some wavelength of light on crystal. Please explain?

Explanation. Sorry, I might use crystallographer slang. I meant an X-ray source with the copper target, which is a mixture of three wavelengths, actually A1    1.54056, A2 1.54439, B1    1.39222, but the beta is removed mainly by mirrors in modern MF sources and very weak.

Topic. Can the change in scale factor with frame number be an indication of beam damage/sample degradation?

Explanation. Yes, typically manifests as a consistent trend upwards in the scale factor as the diffracted intensity decreases

Topic. A non merohedrical twin can only be generated by rotation?

Explanation. Not always - sometimes the twin can be described as a mirror operation. Also, a non-centrosymmetric material can twin to look centrosymmetric - a special case of a merohedral twin.

Topic. It is possible to differentiate racemic twins in the reciprocal space?

Explanation. Existence of a racemic twin is not immediately obvious from the diffraction data, but rather is seen in the behaviour of the refinement (especially in the Flack or Parson's parameter). But there is a statistical test (cumulative intensities) that may suggest racemic twinning before structure solution.

Topic. If I have two crystals with the same a and b cell parameters but with different c length and similar composition (probably trigonal or monoclinic platelets) is it possible to describe it as a twin or would it be something else?

Explanation. This sounds like a "multi-crystal" if the two materials are chemically different. There could be other situations, even possibly incommensurate/modulated structures. This would reTopicuire more detail about the material and the diffraction observations.

Topic. I think the R_int was smaller in the integration process

Explanation. Correct

Topic. Is this issue of twining same as some miller indices value missing (hkl) that does not appear on the PXRD peak? For example, a crystal plane of 110 which is h^2 + k^2 + l^2 = 6 but we cannot get the sum of sTopicuare of three integers to be 7 or 15 because the crystal plane is siting above each other, thus the reason the peak on (2 theta degree) axis do not appear?

Explanation. PXRD is essentially a twin of thousands/millions of components; all the intensities at a given resolution are collapsed onto 2theta. But I think you are referring to systematic absences.

Topic. From hkl4 data, applying platon-twin in olex2, can one find a matrix for a non-merohedral twin? My interest in knowing if TWINROTMAT can be used to find a racemic twin?

Explanation. Yes -- you can use either the Olex2 twin finding routine or TWIN ROTMAT to generate the hklf5 format file from within Olex2.

Topic. Why didn't you try to solve the structure from the hklf5 file? Is it possible?

Explanation. It is possible, but there is a lot of error in the overlapped reflections. Before solving the structure there is no model from which to calculate Fc, so we don't know the contribution each component makes to an overlapped reflection. Thus it's usually a better chance of solving the structure using only non-overlapped reflections.

Topic. If the R_int is a lot higher in Olex because of a problem with how the software interprets your data but it's lower in CrysAlis, can you change the R_int so that it matches the value from CrysAlis? Okay thanks! But then is the Olex value also incorrect?

Explanation. At the point of integration, an R_int value is available, but that is 'lost' if you've only go the hklf5 format file.

We *could* simply print the value from the cif_od -- but is that really correct?

Topic. Sometimes when I switch from HKLF 4 to HKLF 5 file, the R_int in Olex2 disappears (no value is shown). what should I do in this case? When I use HKLF 5 file to rfine the structure and create cif file, the "_diffrn_reflns_number " and the "_reflns_number_total" become the same value. Why? How should I solve this problem?

Explanation. R_int in twins is a really difficult concept. We are still not sure how to display these values. R_int of *which* component? We simply don't have the 'traditional' R_int value from the intensity measurement alone.

Topic. This example in Olex the hkl was an hkl4? And you generate inside Olex the hklf5?

Explanation. Yes, initially it was a hkl4. After checking in Olex2 if it was a twin (and apparently it was), Olex2 found the twin matrix and generated a hkl5 file

Topic. Following on for Horst: when I try this Twin tool in Olex2, I usually get the message ERR: Please check 'tools.get_twin_law_from_hklf5'. Is there an easy way around this?

Explanation. This is strange -- and looks like a bug. Can you send me the 'log' file after the error happens?

Topic. Would like to know what we can figure out about our crystal if it is not diffracting to high angles? Physical reasons and if it means that longer exposures can compensate for it?

Explanation. It depends on the reason for the weak diffraction. To some extent increasing exposure times will let you improve the measurement, but the experiment may become unreasonably long. In cases where diffraction is too weak you might still be able to get a low resolution structure or failing that a unit cell. Trying a different crystal may be a better course of action.

TOPIC. What was the difference between omega and phi rotation?

Explanation. Omega and Phi are two different axes of the goniometer. Both can be utilized to collect diffraction data.

TOPIC. What is the difference between omega and phi? Do we change them simultaneously when collecting data?

Explanation. Omega scans are generally preferred for small molecule crystals because they cover a wider range of the Ewald sphere than phi scans. On the other hand, since the phi axis allows for faster rotation speeds, phi scans may be used when measurements can be completed with rotation around only one axis, such as in synchrotron radiation measurements of protein crystals.

TOPIC. Can X-rays causes any changes in the material? if causes then how to avoid it?

Explanation. X-rays are a type of ionizing radiation, which can cause radiation damage in materials/crystals.
One can try to change the wavelength and lower the temperature to limit the mitigation of radiation damage.
Furthermore one can utilize electron for diffraction, as they are very mild in terms of radiation damage.

TOPIC. In a CAP window, you have I/sig and I/sigo. What is the difference between the two?

Explanation. I/sig is the signal to background ratio. I/sig takes care of all measured reflections while I/sigo only takes care of the observed (intense) reflections.

TOPIC. How does differences in Absorption Coefficient effect our experiment?

Explanation. In crystals with high absorption coefficients, absorption prevents the inherent diffraction intensity from being obtained. This particularly degrades crystallographic statistical values and structural analysis results when the crystal exhibits significant shape anisotropy. To resolve this, software processing is performed to correct for the effects of absorption.

TOPIC. What signal-to-noise range makes sense for a valid experiment?

Explanation. The signal-to-noise ratio should be greater than 3 across the entire range up to a resolution of 0.83 Å. In CrysAlisPro, the measurement schedule is set to ensure the signal-to-noise ratio is at least 15 across all resolution ranges as default.

TOPIC. What is the wavelength you use in micro diffraction?

Explanation. Micro diffraction is typically conducted using Cu-radiation due to its superior intensity.

TOPIC. What is the criteria for deciding the number of scan provide for the sample

Explanation. The lower the symmetry, the more scans must be taken to achieve full completeness on datasets.

TOPIC. After checking my crystal with NMR I note it has some impurity like starting material, is it possible I single out the needed crystal when I am measuring the crystal structure

Explanation. Often materials crystallize as pure chemicals and do not form co-crystals or salts with starting materials of the synthesis. You can often separate your product easily under the microscope when selecting crystals.

TOPIC. Is it possible to run for solving if we got to slow number of diffraction spots?

Explanation. Small molecule crystal structures can typically be solved from <60% complete data. A resolution of ~1.2 Angstrom should be present.

TOPIC. Is kappa-phi same with chi-phi goniometer?

Explanation. kappa and chi goniometers have two different geometries. Kappa goniometers allow for higher completeness of data, important when measuring crystals in P1 space group.

TOPIC. What is charge flipping method?

Explanation. Charge flipping is a type of crystal structure solution method. Please refer to literature for a full explanation of the theory and algorithms used.

TOPIC. How exactly does the detector's position affect signal Topicality? I'm referring to WIT, IVCR, etc.

Explanation. The scattering intensity of the crystal is angle dependent. Low angle data is highly intense, high angle data is typically weak.

TOPIC. What is the accepted value of I over sigma?

Explanation. I/sigma value usually should be higher than 3 in all resolution ranges. CrysAlisPro set exposure time so that I/sigma >15 should be obtained as default.

TOPIC. What is the role of braggs law in this concept?

Explanation. Bragg’s law describes where we expect to see diffracted peaks.

TOPIC. From the theta/2theta goniometry-scan, out main intension is to get crystalline peaks, what we get from phi and chi-scan?

Explanation. We typically do not scan theta in single crystal experiments, this is normally done in powder diffractometry. Other scans are done to cover the data we need to measure for complete data.

TOPIC. If we have 2d crystal growth material, is it the same for measuring?

Explanation. Assuming you are talking about materials like graphene, it will depend on the thickness of the material. You will probably find diffraction is too weak if the material is very (atomically) thin and if the material is flexible this could also cause trouble for a normal diffraction experiment.

TOPIC. Can porosity affect the crystallinity of a sample? I have a material with a very good morphology, but it is highly porous, and I don’t obtain any diffraction peaks at all.

Explanation. The ordered part of the crystal is, in porous materials, a fraction of the crystal volume. What’s more if the solvent is completely amorphous, it contributes to the background. This is a challenging situation.

TOPIC. How to identify the crystal beam damage?

Explanation. Diffracted intensity will irreversibly decline as the experiment progresses. You may also see visual changes to the sample, light dark patches or loss of clarity.

TOPIC. What are its applications in thin film research, such as solid solutions used in high-efficiency solar cells?

Explanation. Thin films are not typically handled with single crystal instruments.

TOPIC. Is it a good practice to delete solvent molecule from the solved crystal structure If there is no any value added information with the solvent molecule?

Explanation. It is only something that we do as a last resort. If the solvent can be identified, it should be included. At the very least it should always be reported.

TOPIC. With a highly absorbent, weakly diffracting (tiny crystal) and suffering from beam damage such as lead iodide perovskites, are Molybdenum or Copper more optimal as sources?

Explanation. Difficult but I would favour short wavelength with long exposures IF diffraction can be seen. This will minimise the absorption and likely the beam damage. If diffraction is too weak and the only way it can be seen is with copper, then that is the only choice.

TOPIC. What does a grey contrast under crossed polarizers reveal about crystal Topicuality?

Explanation. Under cross polarizers you are looking for the light to be extinguished as you rotate the crystal. i.e. the crystal goes dim then bright then dim etc. If this is not happening it means you likely don't have a single crystal. It could be a twinned sample or it might not be crystalline at all e.g. glass. Generally twins will be bright and not go dim, amorphous materials might be dim all the time.

TOPIC. What is mosicity actually!?

Explanation. You will find some good illustrations when searching for images of mosaicity -- blocks of 'perfect' tiny crystals that are arranged 'imperfectly' to form the actual macroscopic crystal.

TOPIC. I have some crystals that are unstable to any oil we're using for mounting crystals. For now, I'm placing the crystals on the microscope slide using a solvent and then mounting it on a loop without oil. Is there a better way to handle this type of crystals?

Explanation. You could use a polar solvent or material in that case. Try using water, ethanol, glycerine or PEG in that case.

TOPIC. I am using vacuum dry to remove the solvent inside the glove box. Is this effect the crystal formation?

Explanation. While vacuum causes solvent evaporation and can precipitate crystals, applying vacuum is usually too fast and yields poorer Topicuality crystals. You may experience different results depending on your chemistry.

TOPIC. Is smaller the size, better the crystal in case of slow evaporation?

Explanation. Generally smaller crystals exhibit better Topicuality. Smaller crystals however may not diffract to high angles for a given diffractometer setup.

TOPIC. When working with air or moisture-sensitive compounds, are there particular crystallization techniTopicues or solvent systems that you’ve found to be more reliable?

Explanation. In my case, I used layering techniTopicues in the schlenk tube with solvent mixtures MeOH-Et2O, CHCl3-MeOH, and also only MeOH for all the layers.

TOPIC. In my case, my chemistry is sensitive to water and air. I used layering techniTopicue for my crystallisation process. Every time it's crystallise as crystalline powder or very tiny crystals. They are not enough to diffract well. Do anyone know how to improve the crystal size and Topicuality for air and water sensitive compounds with any other methods?

Explanation. I assume you are working using Schlenk techniTopicues. Suggestion is to add a solvent in which your material is only slightly soluble and crystallize by lowering temperature in the freezer.

TOPIC. My crystal is always look like fluffy I tried different solvent. Any recommendations?

Explanation. It seems your material crystallized too fast. Try to slow down the process or enhance self-repair of the system, e.g. by crystallizing at higher temperature, slow cool down, addition of modulators (MOF), etc.

TOPIC. How do you decide the right concentration of sample when starting a crystal growth experiment?

Explanation. It is difficult to give an Explanation that is applicable for each case, as every system (crystallization method and molecule) is different.

TOPIC. Is it advisable to use beaker for slow evaporation or small vial? Which one will be better?

Explanation. Both. Always put crystallization in a series.

Explanation. You typically want to reduce the surface are of the interface from solvent to air. Therefore, the narrower the vial the slower the evaporation and the better the crystal, but this is not always true, or necessary. People often find NMR tubes are good for crystallisation for example.

TOPIC. I would like to know about crystallization during heating curve, i.e. MOF and other hybrid molecules synthesis. Is there any good way to determine the parameter for better crystal growth?

Explanation. Addition of modulators, sometimes in very large amounts, has proven to be effective in MOF crystallization.

TOPIC. Hello, when using crystallisation by slow diffusion in gel, there is any recommendations about how can we improve the Topicuality of our crystals (all my crystals contain twins)

Explanation. Please have a look at recent publication by Bernhard Spingler et al.

TOPIC. What exactly is the metastable zones?

Explanation. A region of the phase diagram where the solution is stable and no crystallisation occurs unless the solution is in some way perturbed, e.g. nucleation sites manually introduced.

TOPIC. How to crystallize hygroscopic or oily compounds?

A. Crystallizing hygroscopic or oily compounds is challenging because they resist forming ordered solid structures. Here are two specialized techniTopicues you can try:
1. Co-Crystallization (using a "Crystallization Chaperone")
2. Crystal Sponge Method

TOPIC. How can we find a good seed for crystallization?

Explanation. A good seed crystal may be a crystal of your crude material. You can also try scratching the glass of your vial to create a surface your crystal to grasp on.
Certainly other materials could be tried as well, but should be crystalline. Some people even crystallize using their own hair! Make sure to not use conditioner before.

TOPIC. When obtaining crystals using the anti-solvent method, is it okay to add the anti-solvent until you see a precipitate beginning to form?

Explanation. In that case you will generate seed crystals very rapidly and might end up with large crystals.
If you see precipitation form, add solvent until precipitate is dissolved and lower temperature of the solution slowly.

TOPIC. I have some crystal but they are not in solvent, how can I use this to check crystal structure. Do I need to just add solvent into it or check the crystal structure like that?

Explanation. You can measure single crystal diffraction data on your crystals.

TOPIC. Is there a H-tube which can use for the crystallization?

Explanation. I have used H-tubes for growth of MOF crystals.

TOPIC. Do you know where we can buy it?

Explanation. They may have been custom made by a glassblower

TOPIC. Tried small molecule crystallization using evaporation method in room temp, it was having crystal in the solution stage but after complete evaporation it becoming flakes, why? how can we overcome this?

Explanation . Don't evaporate to completion, when you have nice crystals in solution stop the evaporation. It is almost always better to store crystals under mother liTopicuor and fish them out for experiments, especially if they are delicate. Some crystals incorporate solvent in their lattice and losing it through over-evaporation destroys the lattice.

May be due to collapse by solvent loss or rapid super saturation at the end of evaporation.

TOPIC. The crystal is unstable and the solvent may cause the crystal structure to collapse. Are there any good methods for detecting such crystals?

Explanation. Collect data at low temperature in crystallographic oil like paraton oil

paraton oil . Try substituting solvent with higher boiling point eTopicuivalent solvent, e.g. dioxane instead of THF. 
Measurement at low temperature is needed to maintain solvent inside the crystals.

TOPIC. regarding diffraction pattern is it always have to be concentric?

Explanation. Diffraction patterns always have some symmetry to them, which arises from the arrangement of the molecules in the crystals.

TOPIC. I tried p-chloranil in ethanol, slow evaporation, but instead of getting needle like crystals i got yellow, flakes?

Explanation. One compound may have several crystal structure (polymorphism) depending on the crystallization conditions.

TOPIC. Any tips to crystallize 'tricky' small compounds that behave like organometallic ones, such as riboflavin-Co2+ complex? They tend to form orange gels.

Explanation. Sometimes it works to try 'in-situ' crystallisation -- try an excess of ligand with a tiny bit of metal (or the other way round). Set up many vials, with various compositions). And of course: water, methanol, ethanol etc.

TOPIC. Can we use during evaporation method (mixture of organic-inorganic) stirring under a temperature or that could affect the crystal growth when cooling occur, because i have noticed that during stirring, the crystal is very small and start to grow while there is adeTopicuate stirring speed but how can i take this crystal without being agglomerated later?

Explanation. It is generally difficult to control the aggregation between crystals, but some additives like surfactants may prevent the aggregation. De-aggregation using sonication is also available when your crystals are mechanically stable.

TOPIC. Is there any way to make highly dense pellets with high purity using solid state route method of highly moisturus and low melting point Compounds such as K2CO3?

Explanation. You may want to look at sample preparation methods for IR spectroscopy. Is melting an option?

I want to make highly dense pellets for electrical measurements.

TOPIC. I was working on dicarboxylic molecule (potential ligand for chiral cages/polymer). But it never crystallised in any solvent/s using any method, heating, pressure bomb, vapour diffusion, anti-solvent. Is there any platform, where we can check the feasibility/energetics of the anticipated cage/polymer crystals? 
Or anything other thing that can predict if any crystals can be formed at all (not Fancy computational studies)?

Explanation. You could try to add a base (e.g. tri-ethylamine) and form a salt. These often form crystals easily.

TOPIC. Is the cubic shape of crystal better than the sphere shape for all the modern diffractometers? I heard, that some years ago the sphere shape was the best for the absorption correction.

Explanation. A cubic (or spherical) crystal will always be beneficial for absorption correction. These days we have powerful routines to correct absorption effects even for plate or needle like crystals.

TOPIC. Hello, what does mosaicity mean? Is there a limit number that should I avoid, and change for another crystal?

Explanation. Imagine a mosaic tiling on the floor. The tiles may not be perfectly aligned slightly off. Now consider the tiles are small groups of unit cells in a crystal. Mosaicity is essentially describing how perfectly the tiles line up.

TOPIC. I have crystal sample which grow nicely in freezer, but melt within 3 mins exposure to R.T. Is there any advice to mount that kind of crystal?

Explanation. In crystallographic oil at low temperature

Move the fridge close to the diffractometer, load the sample in the fridge and Topicuickly mount to the diffractometer at 100K, then measure the crystal.

Yes, fast reflexes/

You can cool crystals during selection under the microscope.

TOPIC. I had a crystal that were very pretty in appearance, they also diffracted very well, but the data was always shit. Why would that happen? I have attempted several crystallisation, and data collection, but it is same shit data despite prettiest crystal, and diffraction pattern?

Explanation. Have you check if it is twin?

Perhaps you are in the wrong symmetry. Double check it is not lower symmetry and twinned. It could also be merohedrally twinned. This makes the diffraction patter seem more symmetrical but unless handled in refinement, your model will be bad. Another possibility, unhandled disorder.

TOPIC. For liTopicuid-liTopicuid diffusion, does the anti-solvent have to be miscible with the mother liTopicuor?

Explanation. Yes, the anti-solvent must be miscible with the mother liTopicuor.

TOPIC. Sometimes host is crystal but after adding guest crystallity is loose. Is there any techniTopicue to prevents from the loss of crystallinity?

Explanation. To prevent the loss of crystallinity upon the guest soaking, some factors are important including guest concentration, solvent dissolving the guest, evaporation speed, and temperature.

TOPIC. What have you found to be the most effective way to improve analyte diffusion into a crystalline sponge when the guest is reluctant to go in?

Explanation. To promote the guest inclusion, raising temperature and guest concentration are effective. However, some class of compounds can never be included due to their chemical properties or molecular sizes.

TOPIC. Why larger detector area is not always better for better data collection?

Explanation. The main advantage a large area brings is to measure more data at the same time which allows you to do experiments faster. If you are using the same sensor technology bigger is definitely better, but not all detector sensor technologies are created eTopicual. If you have a large detector but it is noisier than a smaller one, the Topicuality of your data may end up being worse or you may find weak data much harder or impossible to see. To compensate (which won't always help), you need to collect the data several times (redundancy) to improve the Topicuality through statistical merging. In this case though the advantages of the larger area are lost because you have to do collect more data and experiments become longer. In fact, often a smaller detector made which has no noise like our HyPix HPC detectors, it will perform better than larger detectors which are cheaper to make, but have drawbacks i.e. noise.

TOPIC. 1. Is it possible that the molecular configuration observed during the screening process is different from the final refined structure?
For example, could a structure initially appear as S during screening, but after full data collection and refinement, be assigned as R?
2. When using a Mo X-ray source for an organic compound, is it possible that differences in data collection or refinement could affect the assignment of R/S configuration, especially for molecules containing only light atoms?

Explanation . (1) The configuration of stereogenic centres in the molecules typically cannot change upon radiation. At beginning of measurement, only low data completeness is available and the Flack parameter cannot be determined reliably. Towards end of measurement, higher completeness of data is achieved and it may be that the correct handiness of molecule if different from the initial structure solution.
(2) Absolute structure determination of light atom structures (H C N O) should be done using Cu-radiation as the anomalous scattering and intensity is weak when using Mo-radiation.

So, even if I use low-temperature (cryogenic) conditions with a Mo X-ray source, could this still affect the assignment of the R/S configuration?

I have seen a poster in AsCA'25 that claiming even using Ag-source can determine the Absolute Configuration with Flack x -0.03(8), but using big crystal ~200micron, extreme data collection condition of Data Res 0.35Ang with 14-fold redundant.  As the anomalous scattering gets stronger at higher resolution.  Though Cu-Ka would be much easier.

That’s a very good point. To be honest, I am still in the process of learning how to properly handle absolute structure determination, including how and where to refine the Flack parameter.
So far, my main focus has been on data collection strategies for natural product samples, especially optimizing crystal Topicuality, temperature, and completeness.
I understand the importance of the Flack parameter for absolute configuration, but I am still developing hands-on experience in refining and validating it correctly. I would really appreciate any guidance or best practices on this.

Nobody would employ Ag radiation to determine absolute structures of light atom structures. Cu is the way to go, or use electron diffraction. However, It may be a choice in case many heavy elements are present with sufficient anomalous scattering. Pure inorganics maybe!

Sorry I should have mentioned that the poster mentioned was using a crystal of organic compound with oxygen (-rich) atoms the heaviest atom, as a show case.  For my own experience, I have used Mo-Ka (while Cu-Ka is in service) to collect data with good enough Flack x for pure organics, but is Topicuite challenge, not always succeed, big and high Topicuality of crystal is need and carefully planning of data collection.  Cu-Ka is much straight forward.

TOPIC. How or why Ag-target can perform high pressure crystallography?

Explanation. Ag radiation has an even shorter wavelength than Mo radiation. When measuring High Pressure Cells, you want to have short wavelength to achieve more complete data and lower absorption.

TOPIC. Which source may be optimal for really small crystals with big absorption?

Explanation. Absorption coefficients up to ~40-50 mm-1 can be handled with Cu radiation by choosing a small crystal Rule of thumb is crystal size ~ 1/µ . Always choose crystal size according to wavelength/absorption coefficient and characteristics of the X-ray source.

TOPIC. Why short-wavelength source (Mo and Ag) is more suitable for measuring charge density?

Explanation. The shorter the wavelength, the more data can be measured according to Bragg eTopicuation "1/2 lamda".
In charge density analysis, you need to measure many reflections/data points to build up crystal structure model, e.g. in multi pole refinement.

TOPIC. Cu k alpha has a higher wavelength as compared to Mo. In such cases if we have both metals and organic things crystallized together, that time won't that be difficult to resolve C-C bond which has a bond length of 1.54 A while using Cu K alpha?

Explanation. Cu radiation is perfectly suitable for measuring coordination compounds. You can measure data up to ca 0.79 Angstrom, C-C single bond is much longer.

TOPIC. What is preferred to be used in organic-inorganic materials cu tube or Mo one or it depends on the size of molecules?

Explanation. Coordination compounds and MOFs can be measured best with Cu radiation.

TOPIC. If we have some solvents trapped in the lattice and if we collect data both at higher and lower temperatures and we see the space group changing, does it mean that the crystal is going through a phase transition or it’s just the solvents that is creating trouble at different temperatures?

Explanation. A change of space group is a phase transition - just a different word. It may happen that the symmetry of the crystal is changed because molecules/solvent arranges differently inside the crystal

So does it mean for a phase transition the whole structure has to change to call a phase transition? Just a change of the space group doesn't mean a phase transition?

It is enough if the crystal structure changes partially.

TOPIC. Why does the Flack parameter exhibit significant error when determining absolute configuration using Cu radiation? How can this error be reduced?

Explanation. The higher the I/sig(I), the lower is Flack parameter. The higher the redundancy in the dataset, the lower is the error on the Flack parameter.

TOPIC. Is the volume and cell parameters like a,b,c and angles changes/increases at high temperature measurement compare to low temperature.

Explanation. Yes, that is the norm: Unit cell parameters increase with increasing temperature. However, there are also materials with negative thermal expansion coefficient.

TOPIC. Is there a system with two radiation sources, for example, with two wavelengths, that we could use to remove fluorescence effects from some samples?

Explanation. Yes, absolutely! XtaLAB Synergy platform allows to integrate two different wavelengths. Furthermore, using hybrid photon counting technology (HyPix) you can suppress the fluorescence signal picked up by the detector.

TOPIC. What type of glue can be used to fix the crystal to the loop? When should I use glue instead of just oil?

Explanation. Normally I would use glue if you: a) want to permanently keep the crystal so it cannot slip or move or b) are performing room temperature or higher measurements in which cases the oil can be less viscous and not hold the crystal securely enough. As for the type of glue it depends a little on your use case, superglue (cyanoacrylate) or epoxy glue.

TOPIC. In a case where your crystal moves from the loop, is it possible to observe this on your screen (after few frames) during the data collection?

Explanation. We have some tools that can identify this in the data processing and data analysis side of things. There is an option on the lattice wizard "check for the sample jumping" which tries to find any sudden discontinuous changes in the diffraction pattern. The data processing has a similar option to recover the experiment by considering this type of change in the diffraction pattern

TOPIC. I have microporous materials, which are usually weak diffracting samples, so I usually mount larger crystals to increase a bit the diffractions. Is this wrong? Should I focus on smaller crystals?

Explanation. It all depends on absorption coefficient of the sample. You can certainly take a little larger crystal than the size of X-ray beam to increase scattering intensity in most cases.

TOPIC. I must do my measurement in sealed capillary and it will be pressurized with gas (so not to thin). Are there procedures to account for absorption correction from cylindrical shape of capillary on top of "drown" crystal shape?

Explanation. The absorption correction we perform is called multi-scan. It uses symmetry eTopicuivalents to determine the changes in absorption as the crystal rotates in the beam. This approach is normally Topicuite good at compensating for a wide range of cases like this where you have little choice. It cannot correct for data which is too weak to be observed of course. You can also (if you can see it) face index the crystal - i.e. describe its shape - which can help absorption correction algorithms by providing knowledge of how absorption physically should be. I assume this latter part is what you mean by "drown" crystal shape. Additionally, CrysAlisPro interfaces with Ross Angel's ABSORB program which has some very powerful additional tools for absorption correction.

Indeed- should be "drawn faces". CAP allows import of data (frames) from beamlines/other vendors (sometimes better than native programs)- Is it possible to also add "crystal movie" and process that for absorption correction?

That is more difficult, we need to know the microscope position and movie format etc. Because there is not 'standard' for this, it is not easy to support every combination and so we generally don't support those which are not Rigaku configurations.

TOPIC. u >80 mm' recommended to switch to longer wavelength X-ray source. Why not shorter wavelength, I think absorption shall be reduced?

Explanation. Typically, a shorter wavelength has a lower absorption. It depends on the absorption edges of the material.

TOPIC. Yes, If my sample has a μ larger than 80 mm-1 under Mo , that means I need to try Cu？

Explanation. if your sample has µ=80mm-1 with Mo, you need to take a small crystal. Maximum size is ~1/80 mm ~ 15 µm !

TOPIC. Absorption coefficient with Cu should be much higher.

Explanation. Yes, This is what I mean, I think this sentence “u >80 mm' recommended to switch to shorter wavelength X-ray source.

TOPIC. Once get a crystal of my sample but I never thought I will need to check the crystal structure so I filter off the solvent of crystallisation and keep the crystal in solid state. But I'm trying to recrystallized it again because I needed the structure unfortunately I can't get that done now. My Topic is how I can use the crystal that I have which is not in solution for crystal structure checking?

Explanation. If you have a crystal already, then measure a single crystal diffraction dataset out of it.

I see. You have crystallised, dried the sample and now it is the only thing you have to work with. Well, you can still try to mount it onto the diffractometer with a loop and some mounting oil. If it doesn't diffract you are probably stuck. Your options are very limited, use a more powerful instrument than you have, use Cu radiation if you aren't already, try a synchrotron or you could crush it and use it for electron diffraction, if you have access to an electron diffractometer but they are Topicuite a recent development and there aren't many around.

TOPIC. Larger area CMOS/CPAD detector and smaller area HPC Detector work similar in terms of data collection? As the HPC will have less noise than CMOS detectors?

Explanation. CMOS/CPAD and HPC detector have fundamental technological differences. You are correct that HPC is a noise free detector and CMOS will always have noise. HPC detector can be arranged in a curve around the sample to achieve a high theta coverage and shorten measurement times and improve data Topicuality even further.

TOPIC. Absorption coefficient is around 150 mm-1 for complex intermetallic. Those kind of crystal what you recommend and how to the issue

Explanation. This situation favours short wavelength Mo or even Ag, and a very small crystal. You also want the crystal to be as close to spherical as possible so e.g. cubes would be better than needles. Cutting the crystal can help make it more symmetrical or ideal, grinding into a sphere which is Topicuite difficult and uncommon these days. You should definitely face index this sample for best results.

TOPIC. We are selecting 20-micron size around, It is difficult to do face indexing and also we have only Mo facility only. Is there any other parameter correction will helpful? Do you recommend any specific strategy data collection?

Explanation. For strategy, collect as much data as possible. More redundancy will help the multi-scan algorithm work as well as possible. You can try adjusting the spherical harmonics by hand in the refinaliser or eTopicuivalent stage for your vendor. You could also consider a smaller sample. It may still diffract well. If you can make its shape as symmetrical as possible, avoid rods, or needles, stick to cube type geometry or spherical. Try the Gaussian correction with beam profile enabled. It might also help.

TOPIC. About sample centering if we use automatic Gonio head, will the sample be centred automatically. the video showed was about the manual Gonio and centering the sample manually? if yes, why a manual Gonio head is used anyways?

Explanation. Yes, motorized goniometer heads can centre the sample automatically using optical image recognition or X-ray based centring. The reason manual goniometer heads exist is because they are more common, cheaper and still effective. Motorized heads are more expensive, a more recent development and they are bulkier which may affect strategy efficiency.

TOPIC. Why do you sometimes see it is a 'well diffracting sample' but see a very few diffraction spots on the screen? Should you rely on it or always look for the sample showing a high number of diffraction spots?

Explanation. This sometimes happens to me too. I usually try another crystal with more diffraction spots. If the diffraction spots are few and at low angle, then it will be difficult to get a reliable structure and absolute configuration

TOPIC. Is it safe to collect data overnight at room temperature? Would this pose any risk to the detector?
For context, the laboratory temperature is maintained at 22–25 °C with relative humidity around 36–45%.

Explanation. I have collected data for more than 1 week at room temperature and it was fine.

Concurrent single crystal diffractometers do not need constant attention and supervision. Data collections can indeed run multiple days, e.g. when running a charge density data collection.

TOPIC. Regarding closing the slits for twining crystals, Is there an ideal intensity value or should I play around until see the peak separation?

Explanation. The advantage of a continuous slit is that you can close it just enough to achieve separation without sacrificing too much beam intensity. You should close only just enough to separate if your sample is weak. If it is strongly diffracting, close it enough to separate but don't worry too much about the intensity as long as data Topicuality is sufficient

TOPIC. Can we change the target resolution = 0.837 before start experiment?

Explanation. Yes! You can change preferred parameters for the measurement on the fly

TOPIC. How to calculate the absorption coefficient of a material? Could you tell me for an example like TiO2?

Explanation. CrysAlisPro can calculate µ or you use external programs

TOPIC. I had instances where I change the strategies to have I/sigma and other parameters (individual I/sigma, merged I/sigma) and different thetas (because I read the value of 30 is usually good), but the data collection time changes without significantly changing the completeness. How does this work, what do the numbers there mean?

Explanation. You may have missed to recalculate the strategy, or you measured with the wrong point group settings.

TOPIC. I would like to determine the absolute configuration of the compound; should the checkbox at this position （—Friedel mated are eTopicuivalent）be checked or unchecked？

Explanation. It should be unchecked. The information about absolute structure is contained in very small differences between the Friedel pairs. When we say all diffraction patterns are centrosymmetric, this is largely true except for these slight differences in the case of chiral materials.

TOPIC. If we increase the exposure time, is it possible to increase the resolution? What might be the other way?

Explanation. For a given crystal, crystal orientation and diffractometer, increasing the exposure time is the only way to increase the resolution.

TOPIC. The dots we see in the screen are indicating single molecule or atoms in a single molecule?

Explanation. *All* atoms contribute to *all* dots. The *position* of the dots is determined *only* by the unit cell dimensions (a, b, c, alpha, beta, gamma). The intensity of the spots is determined by the contents of the unit cell (or rather, the asymmetric unit). Often, the asymmetric unit contains a single molecule.

TOPIC. So, 1 dot is a reflection of all atoms and indicate whole molecule. Is it wright?

Explanation. The dots are bragg peaks (we also call these reflections) The position of the bragg peaks tell you the shape of the unit cell and its dimensions. The intensity of the peaks tells you about the position of atoms within the unit cell. You need to do a bit of computation to get from the intensities to the atomic coordinates.

TOPIC. What is the maximum permissible displacement (defined by distance standard deviation) for sample wobble? I wonder if the SD tolerance range also applies to macromolecules.

Explanation. First sample wobble is describing how badly the sample has been centred. I do not know the upper limits I'm afraid, but clearly if the numbers are large, you have performed your experiment poorly. You should seek to centre the crystal better and repeat (if it is preventing you from solving the structure.)

TOPIC. In many cases, I have seen the reflection at rotational axis continues to multiple frames. What is the reason behind this?

Explanation. Reflections close to the rotation axis are in the diffracting condition for longer. We apply corrections (Lorentz polarization) to scale them. Very close reflections are typically unreliable and rejected because they corrections need to be very strong.

Explanation. I think, In single-crystal XRD, a reflection far from the rotation axis intersects the Ewald sphere only briefly and therefore appears in one or two frames, whereas a reflection on or very close to the rotation axis remains near the Ewald sphere over a wide rotation range and conseTopicuently appears in many consecutive frames.

TOPIC. Is it going like this?

Explanation. Yes.

TOPIC. If the radius of the Ewald sphere increases by using a lower wavelength, then why do we say the Mo source gives lower resolution than a Cu x-ray tube source?

Explanation. You can collect to *higher* resolution with Mo than you can with Cu.

Yes, the intensity of Mo radiation is less than that of Cu (all else being eTopicual) -- but there is a resolution limit for both radiations (due to the wavelength). You *can* get to higher resolution with Mo, but you may need longer exposure times to get there.

TOPIC. Should we use the filters that are suggested (e.g. for P-lattice, F-lattice etc.)?

Explanation. Generally, yes. They will be properly activated depending on the crystal system you have from the indexing. The proper approach if wrong is to reindex the sample to find the correct unit cell. I cannot think of an occasion where I needed to manually change the filter or that it was helpful to do so.

TOPIC. Is it safe and to what extent can one omit out bad reflections (what most crystallographers regard as junk) from the reciprocal space?

Explanation . By 'bad reflections' you probably mean reflections where their *measured* intensity does not match that of the *prediction* from your model. If your model is essentially correct (i.e. its predicted reflections generally match the measured values) and you then find that there are *few* bad reflections, then these are most probably 'bad' because of an error in the measurement -- they might have been under the beam stop, for example, or they might appear stronger than they should do because a cosmic ray happened to hit your screen just where the reflection was on the frame.

Of course, a proper beam-stop mask will make sure that the regions in space that are behind the mask will not be taken into account.

If you have *lots* of bad reflections, this could point to a systematic problem -- either experimentally (crystal or instrument not cantered/aligned), it could point to a problem with the crystal (twinning, for example) -- and it could be due to highly anisotropic absorption etc.

TOPIC. Sometimes the diffraction spots in 3D Ewald explorer are generally well aligned, but I also observe some split reflections or weak spots around the main reflections, but it does not look like a typical twinning, since there is no clear second, well-aligned lattice visible. What could be the reason for these extra diffraction spots? Also, what would be the best way to handle or exclude these reflections during data processing or experiment?

Explanation. First comment, if reflections are there then their cause should be understood before deciding to dismiss them. That said possible reasons from the top of my head:  1. you have mounted 2 crystals (passenger) 2. You have a cracked crystal 3. You have lambda/2 diffraction 4. You have superstructure (commensurate or incommensurate) reflections.

TOPIC. When you click "Follow sudden changes of sample orientation" what values should you put in the orientation search and the search steps?

Explanation. Normally the defaults are suitable. If they are not, you can try increasing the search range. I would normally leave the steps alone

TOPIC. Normally the defaults are suitable. If they are not, you can try increasing the search range. I would normally leave the steps alone

Explanation. Like SADABS used for absorption correction, how I can use absorption correction in CrysAlis

During data finalization you can apply absorption correction harshit

TOPIC. what is the meaning of complete sphere and full sphere please explain it?

Explanation. A full sphere is basically everything you can possible measure. Every accessible HKL index is collected. Complete sphere means you have collected them all.

TOPIC. Modelling disorder around multiple symmetry elements, for example, a mirror plane and a 2-fold axis within the C2/c space group. I appreciate if you can demonstrate it briefly.

Please show us how to add H atoms manually on olex2, i mean using commands.

Explanation. Use HFIX in Mode menu on the top of the Olex2 window, select proper hydrogen type, and click atoms you want to add H atoms.

To refine a model around symmetry element, which connects to itself as a result of symmetry generation, assign a negative PART number to avoid the self-connection.

TOPIC. 1. If a customer does not want to share the molecular structure model because the work has not yet been published, is it still possible to perform SC-XRD refinement?
How is this typically handled in a service crystallography context?
2. During refinement, is it possible for an initially assigned R configuration to change to S (or vice versa)?
Under what circumstances can this occur (e.g. data Topicuality, anomalous scattering, and refinement strategy)?

Explanation. (1) The electron density map should reveal what the molecular structure is. There is technically no need to have a chemical drawing ready before doing crystallography, but it certainly is always helpful to have a rough idea what you are dealing with. Knowledge about the expected elements, solvents used during crystallization, precursors etc. are all valuable information.
(2) It certainly happens freTopicuently, that the initial assignment of R- and S- are not correct in the first steps of structure refinement. You can invert the structure model with command "inv -f"

TOPIC. how to fix solvent disorder or any alkyl chain disorder

Explanation. To fix disorders of highly flexible solvents or alkyl chains, use of geometrical restraints like DFIX and DANG will be helpful.

TOPIC. Do we need to install ShelexTL or any other software before installing Olex2?

Explanation. Olex2 can operate with Shelx programs but also comes with its own structure solution and refinement program.

TOPIC. for example, i am aiming to make M-N-C type catalyst for its utilization against CO2 electrochemical reduction. I do use X ray absorption fluorescence techniTopicue (both in situ and ex-situ) to get to know about the structure type (symmetric/asymmetric), oxidation state of metal, coordination number, interatomic distances to know metal is attached with M/C/O. So the Topic is can we get the same type of information here?

Explanation. Yes, single crystal X-ray analysis gives structure symmetry, coordination number, and interatomic distances as a shape of the molecule. From these information’s, you can also speculate the oxidation state of metals. However, SCX needs sample crystals, so in-situ measurement reTopicuires some techniTopicues to crystallize the state.

TOPIC. Sometimes due to symmetry I think, the carbons in a phenyl ring are not all visible, so I can't use afix 66 and I use dfix and dang to get it right, is this okay? Also, I have some cases where the phenyl rings are "cut" in the middle (and only a part of it is visible) and only when I use command grow (or control + G) the phenyl ring is formed. How can I fix the aromaticity of this type of rings when not all of it is visible in my asu.

Explanation. Just fix the aromaticity on the atoms you can see and the ones you cannot see (ctrl+G) will be fixed automatically by Olex, as they are related by symmetry.

It sounds like you are seeing phenyl rings on special positions. You can model these using 'PART -1' and 50% (or appropriate) occupancy.

TOPIC. If you can see TOPIC-peaks for hydrogens, is it better to change the TOPIC-peak to hydrogen atoms or just add hydrogens geometrically?

Explanation. Both ways are acceptable, but the latter is usually used. This is because contribution of hydrogen to the diffraction intensity is very small, and the positions of the electron peak and the hydrogen nuclei are slightly different due to the withdrawal of the bonding electrons by the mother atoms (C, N, O, etc).

TOPIC. What about the shift? How can you improve it?

Explanation. By giving extra refinement cycles

It’s not always the case if the structure doesn't converge.

"Shift" can correspond to all parameters that you refine. E.g. atom positions, ADPs, extinction etc.
Eventually these parameters cannot be further refined, then the shift is zero.
If you have convergence problems, than you need to improve the model.

TOPIC. If you get a really good fitting of Fobs vs Fcalc despite not having good I/sigma statistics can we still assert the crystallographic model as correct?

Explanation. That depends on whether you meet IUCr guidelines or understand and can justify why you could not. Trying to refine against noise is not sensible and if your data is weak, there is more chance that you are forcing your expectation on the data, rather than letting the data speak for itself.

TOPIC. How olex2 bypass sTopicueeze works and should we have platon installed or linked with olex2 before doing solvent masking using bypass sTopicueeze or both works separately?

Explanation. Olex2 has its own algorithm. The techniTopicue originated in Platon. There are two options for achieving the same result. BUT I would prefer Olex2 as it is more streamlined and does not modify the original data file. The algorithm is trying to calculate the contribution of the 'missing'/'empty' parts of your model to the reflection file and subtract them out. It is possible to seriously bias your structure using this techniTopicue so be careful and describe everything you do and why. Use it sparingly.

TOPIC. I had instances where the solution only works if solved in olex-2 solve, if any other method is used like ShelXT or ShelXS, it doesn’t solve it well. Why is this so?

Explanation. We have different structure solution programs because one may work better for a particular material than the other. ShelXT works great for organics and coordination compounds. ShelXS works well if at least one heavy element is inside the crystal structure, e.g. inorganics. ShelXD for large organic molecules etc. Olex2.solve is another great one, can be used for many types of materials.

TOPIC. What to do when there are no TOPIC peaks around the atoms, however when the electron density Maps show there is something wrong around Cl and F atoms in a symmetric molecule? I have tried to fit all atoms possible, but still seems that something is wrong in the electron density maps. Could it be an occupational disorder? How to fix it?

Explanation. It may be a site disorder, where Cl and F atoms occupy the same positions as a disorder. Use EXYZ and EADP constraints to fix the Cl and F atoms at the same position, distribute their occupancies so that their sum should be 100%, and refine.

Thanks! Should the atoms occupy 50% in each position? Or it is freely refined?

Without special reasons, free refinement is recommended.

If they are freely, the molecular formula seems weird. There is a way to edit it in Olex? In the cif file the formula should appear the as it is, I mean with point numbers in Cl/F, or can I consider whole number?

TOPIC. In my structures I have strong Topic-peaks located very close to heavy atoms (Bi) and some in between bonds of heavy atoms. These values are around 8-5 in the Olex scale. I tend to chalk this up to heavy atoms being finicky but perhaps you can offer some insight into why this might happen. I am very certain that these Topic-peaks cannot be from foreign atoms. I am somewhat certain that they are not due to disorder since giving them a partial occupancy does not lead to an improved R1. For context this is a layered inorganic crystal with +/-2 charged layers, not a small molecule.

Explanation. Did you consider careful absorption correction and choice of an appropriately sized crystal? It sounds your crystal was too large and you measured too small redundancy to employ empirical corrections effectively.

TOPIC. In the Ctrl+M (Maps) mode, what do the green and red spheres mean? Does the red mean bad residual electronic density?

Explanation. If you observe a large red residual electron density around an atom, it indicates that the atom you assigned is too heavy, and a lighter atom should be used to match the observed electron density. Conversely, more green density suggests that the assigned atom is too light, and an atom with a higher atomic number should be chosen to match the observed electron density values.

TOPIC. Should we handle the disorder first and then add hydrogen atoms? Also, is it fine to deposit cif file that has some atoms without thermal anisotropies?

Explanation. It depends case to case, but usually it is easier to assign hydrogen atoms after handling the disorder of the atoms they are attached to first,
Yes, sometimes you will not have enough parameters to describe all atoms with anisotropic displacement parameters. You will need to explain this in your checkCIF file before structure deposition.

TOPIC. Why we lost the phases?

Explanation. When an electromagnetic wave is measured, only the sTopicuare of the function is recorded. This erases the phase information. The only way to maintain phase information is to make use of a techniTopicue like holography.

Yes, we can measure magnitude directly, not phase. We need to calculate what the phases were that gave rise to the magnitude. That is the whole goal of structure solution.

TOPIC. How to solve and refine a structure where you have on an electron density two different type of atoms? For example, there are two different metal ions on the same electron density

Explanation. If two types of atoms occupy the same location in the crystal, please fix them at the position using EXYZ and EADP constraints.

TOPIC. When both split disorder and positional disorder are present, unreasonable bond distances at that time how to fix the bonding distance

Explanation. When the bond distances got unreasonable as a result of disorder treatment, use of geometrical restraints like DFIX and DANG will be helpful.

TOPIC. Is it possible to fix a specific space group for a shelxt solution? For example, if I know that the unit cell should be centrosymmetric but the software gives me a chiral space group, are there command line options to force an achiral solution?

Explanation. You can force ShelXT to use a specific space group with -s"SPACEGROUP"   e.g. -s"P2(1)_c" or -s"P-1" etc.

TOPIC. Sometimes the atom shows a negative value. I forgot is it called ndc? (or sometime like Negative real dispersion correction) Can you teach me how to resolve it?

Explanation. If you have an npd, it usually means you have the wrong atom on that site, or there is disorder or absorption is not properly corrected. There may be other causes too. You need to correct the model and/or ensure data collection and processing has been done well.

What if I am sure that is the right atom? Then how can I resolve the disorder issue?

Difficult to Explanation without seeing the structure. Each structure is a fairly uniTopicue puzzle to be solved.

TOPIC. Can a PCR file from full proof program can be used in Olex

Explanation. No, Olex2 cannot read PCR format.

TOPIC. Sometimes we have more than one kind of solvents sharing common sites and disordered about the symmetry and even sometimes it’s not possible to know the exact nature of solvents present. Can we use mask in that case?

Explanation. You can assign multiple solvents/molecules/ions per void.

TOPIC. I mean how to justify that what we have masked?

Explanation. If a molecule cannot be modelled, than usage of solvent mask is fine. Best crystallographic practice is to use model the solvent, in some cases it is simply not possible.

TOPIC. In which instances do you change the occupancy of certain atoms?

Explanation. Right-click the atom, and use "chemical occupancy" to set a fixed occupancy.

TOPIC. What will you suggest if only some solvent molecules can be welly located but others are in a mess.   Do sTopicueeze for the rest of the mess or just erase all solvent molecules and sTopicueeze?  How about ions?  We better not erase ions no matter how bad it is, right?

Explanation. Good practice is to model the molecules that you can really see and model. The others that only give diffuse electron density can be masked.
Certainly you can also mask ion, e.g. large soft ones like NEt4+ or there like.

TOPIC. What to do if you are not sure regarding the solvent? Could it be as "unknown solvate"? Or would be better do not use mask at all?

Explanation. You should have other evidence. e.g. the synthetic chemist knows what solvents were used, recrystallization solvent should be known. Use the electron counts from the solvent mask to work out which (or which combination) might be present. You may need to make a judgement.

TOPIC. Does olex2 software have any parallelisms with Reitveld refinement?

Explanation. Rietveld refinement fits a calculated diffraction pattern to powder data, refining an average crystal structure from overlapping reflections, whereas single-crystal refinement uses individual, resolved reflections from a single crystal to determine a much more detailed and precise atomic structure, including anisotropic displacement parameters and subtle disorder. Olex2 is a single crystal refinement suite.

TOPIC. For the NoSpherA2 option, if the structure is not like a molecular crystal, how to use this option?

Explanation. NoSpherA2 should not care. It is using non-spherical form factors that are calculated from your geometry.

TOPIC. Can you explain the systematic absence plot a little? Sometimes we are getting blank window.

Explanation. The plot shows reflections which violate the expected systematic absences and their intensity. These should be weak or it may indicate you are in the wrong space group or something else is going wrong. If you are getting a blank window, maybe you are in a space group which does not have any symmetry elements that cause systematic absences.

TOPIC. What is SWAT?

Explanation. Solvent water correction. It is a method of modelling diffuse solvent (Moews & Kretsinger, 1975)

TOPIC. Is there a way to generate the crystallographic info as a table for the Supporting Information for journals? Thank you!

Explanation. Olex2 has the 'report' function -- where you can get a basic report that you can cut/paste. There's also the more sophisticated ReportPlus extension for Olex2, which makes complete reports

TOPIC. When merging data from "twins"/multiple domains- is R_int still valid?

Explanation. Absolutely. After merging a new R_int is calculated based on the merged data.

TOPIC. How to open the file in CrysAlisPro?

Explanation. Generally, datasets can be open by double clicking on either the *.par or *.run files. Please let me know which file you referred to a different file.

When I open the .par or .run file it gives a diffraction pattern how to make structure from this. which command to tool i have to use for the same

TOPIC. Many of my samples are medium-sized organic molecule (around 10kDa) and the crystals were often grown from water. And they often diffract to 1.0 Angs. Can they be published as you mentioned that the high resolution should be better than 0.84 Angs? The R factors are also high around 15% despite best attempts in refinement. Can they be accepted by IUCr standards?

Explanation. The 'rules' change for anything by 'small-molecule'. You are talking in kDa, so this tells me that these are not small-molecules!

The molecules you describe should contain a vast amount of solvent/water. These type of crystals often have a resolution limit around 1.0 Angstrom, similar to porous materials (MOF, COF, etc.) even when measured with strong rotating anode sources or at synchrotrons. 
Definitely such data is publishable, but with R1~15% it sounds you did not treat the disordered solvent correctly. Please refer to Horsts presentation on solvent masking.

TOPIC. I would like to ask for clarification regarding CCDC data policy.
Is there any expiration or time limit for CIF files that have been deposited to CCDC but are kept on hold (not yet published)?
I am asking this to ensure I provide accurate information to users/customers.

Explanation. You best ask this to the folks at CCDC directly.

TOPIC. Is it possible to Topicuickly get CCDC numbers via OLEX2 ?

Explanation. There is a 'get ccdc number' button in Work > Report, but I am not sure whether this still works on the CCDC side. I think they want you go through their portal instead.

Got it. It seems that the users need to log in, deposit cifs and get numbers through their web platform

TOPIC. When i open the .par or .run file it gives a diffraction pattern how to make structure from this. which command to tool i have to use for the same.

Explanation. you can go through them

You need to ask your local crystallographer to do so.

You can conduct structure solution and refinement in the program of your choice (we recommend strongly Olex2 !). These programs have a direct interface to CrysAlisPro.

TOPIC. Is measurement of density expected by CCDC or density only from refinement sufficient?

Explanation. You do not need to fill in all available entries of your CIF file, but it is best practice to submit as much as possible.

TOPIC. Usually I only address Alerts A and B. I usually ignore alerts C, is that okay?

Explanation. Not really -- alerts mean that that there is some sort of issue with the structure -- ideally, please look at all of them. If you can't solve the issue, please try and explain what the problem is and why it can't be solved.

TOPIC. I can't install "Platon" on my OLex2? How can I install it?

Explanation. Install Platon the normal way, and then let Olex2 know where it is by including the folder where it is installed in Home > Settings > Path

TOPIC. I think CrysAlisPro did a good job to prevent the loss of data at the low angle region due to overexposure by recollecting data with attenuation, while the losing of such data could cause Alert B sometimes.  
It that always true though?

Explanation. Data recollection was needed in the era of CCD and CMOS detectors. Concurrent HPC technology does not reTopicuire re-measurement of overflowed reflections due to the superior count rate and counter depth.

So yes, if you have overflow reflections, then they should be re-measured with shorter exposure time.

TOPIC. How could I justify this alert B: PLAT417_ALERT_2_B Short Inter D-H..H-D       H1       ..H3D      .       1.97 Ang.  x,y,z  =      1_555 Check. I have tried to fix the position of H, but at refinement cicle it moves a little bit.

Explanation. Due to misplaced hydrogen atoms

You need to fix this. This means that H1 and H3d *in a neighbouring* molecule clash. In Olex2, right-click on H1 and then choose BANG (Bonds and Angles) to 'grow' the arrangement.

If you do BANG, you can see the two hydrogens that are close and the use fit to move one of the hydrogens away

TOPIC. When I get a Hirsfeld alert I usually use simu to get rid of the alert, is that okay?

Explanation. you should try and figure out *why* there is a problem.

Usage of SIMU, DELU, RIGU can be justifiable case by case e.g. if the large ADPs are not due to split atom positions etc.

Yes, that is assuming I have ruled out he case of disorder. Thank you!

TOPIC. During distortion in phase change at higher temperature, can the model be resolved to identify structure change at this temp

Explanation. If the phase transition results in another crystalline phase it should be possible to solve and refine the crystal structure at the elevated temperature.

TOPIC. Can a wrong space group cause to have that triangle in the structure?

Explanation. Yes, wrong space group can cause the negative ADPs. Please try to check other possible space groups.

TOPIC. Is this value of shift can be published in acta B ? (with disordered structure and twining?)

Explanation. Shift should be zero, maybe by fixing a certain parameter. If shoft still remains, then data or structure model must be revised.

TOPIC. Could you briefly explain what is Hirshfelf Test Diff?

Explanation. The components of the anisotropic displacement parameters along chemical   
 bonds are assumed to be eTopicual in magnitude. Large differences might
 indicate contamination of these parameters with other (unresolved) effects 
 such as (substitutional) disorder, model or data errors and/or 
 over-refinement.

TOPIC. May I ask how to show or what number shown ADP in Olex2?

Explanation. Right click the background of Olex2, and select "Draw style" > "Ellipses & sticks".

TOPIC. When depositing a CIF, should a disordered site be uploaded explicitly as a disorder model (e.g., two atoms with partial occupancies), or is it acceptable/preferred to represent it as a single atom?

Explanation. You only deposit a single CIF file. That file contains information of the disordered moieties as well.

I understand you are trying to ask how to choose between modelling as a single atom with a large ADP or 2 disordered parts. It depends on how big that single atom is. I would also choose disordered parts when it described the chemical connectivity better and crucially refines stably. At some point you have to make a judgement on how big is too big. Also you should keep an eye on the data to parameter ratio. Obviously modelling disorder will increase the number of parameters you are modelling. You need to find the right balance.

TOPIC. Sometimes we are getting alerts on solvent accessible voids even there is no significant electron density left to be model

Explanation. Yes, that happens.

TOPIC. Is that means natural empty space in lattice gob ?

Explanation. Use solvent mask in that case. Interpretation must be done though.

You should write a comment for this alert. This could be due to desolvated solvate. You can find examples of comments for this sort of alert online from already published structures

TOPIC. How does MicroED compare to an atomprobe?

Explanation. MicroED is a diffraction techniTopicue, whereas an atom probe is a spectrometry techniTopicue. During the analysis the crystallographer works out the arrangement of the atoms in the crystal by modelling.

Thank you, so these are two different approaches for a similar study/outcome...?

TOPIC. Can we determine the absolute stereochemistry of a molecule without any heavy atoms (e.g. only C, H, N, O) using microED??

Explanation. Yes.

TOPIC. For powder samples in micro ED can you get the structure from grain by grain?

Explanation. microED is a single crystal techniTopicue. the sample is what we normally call a powder but we do experiments on individual grains which are just very, very small single crystals. the data we get is like a single crystal X-ray experiment not a powder experiment

TOPIC. It seems the sample we need is actually what we use for powder XRD.  Then, will the size of the grains affect the result?  Still bigger the better or actually smaller is better (though typical size <100nm right?)? and more uniform the better or it doesn't matter.

Explanation. 3DED has an upper limit before absorption of the beam becomes intolerable. It is a balance. A crystal which is not too big, not too small. Thicker grains will suffer more dynamical effects (in general) so thinner is better from that perspective, but too thin may not diffract well enough.

TOPIC. Is Rigaku XtaLAB Synergy i capable of running this analysis or another instrument is needed?

Explanation. No. XtaLAB Synergy-ED is our only electron diffraction instrument. The reTopicuirement are very different from X-ray. XtaLAB Synergy-i, S, R, DW are all X-ray instruments.

TOPIC. How are the vacuum-sensitive samples (porous MOFs with lattice solvents trapped inside) protected during ED measurements?

Explanation. Cryogenic sample holder for TEM is available for protection of such samples.

TOPIC. Is it possible to compare MicroED and Precession Electron Diffraction?

Explanation. MicroED moves the crystal by rotation with a fixed beam, precession moves the beam by precession with a fixed crystal. You can get structures from both but the data you get is processed differently.

Precession (PEDT) reTopicuires a higher dose and may not be ideal for radiation sensitive materials. MicroED works typically on doses < 1 e*A2

If you do a tomographic series (discrete step-wise rotations) with precession ED (PEDT), you will get a very similar kind of dataset which you can likewise use for structure determination, though data reduction will need dedicated packages (like PETS2 or ADT3D). Now a days, continuous-rotation MicroED is used more often due to it speed and lower radiation exposure, but also PEDT has its advantages, e.g., it is great at cutting out tiny crystals within agglomerates or amorphous regions.

TOPIC. Hello! Is the refinement similar to data from XRD? Regarding the R_int, generally is it higher?

Explanation. The procedure is virtually identical to single crystal X-ray diffraction (and very different to PXRD). We use different scattering factors that better describe electron diffraction. In general R-factors are higher than we are used to from X-ray diffraction.

Yes, all R-factors become higher because of the dynamical effects making each intensity different from the sTopicuared structure factors, and also to those that would be eTopicuivalent from symmetry alone. In refinement, this can be modelled, which will decrease R1, wR2, etc., ideally back to XRD level. On the other hand, during the data reduction, where you would investigate internal metrics like R_int (or rather Rrim etc.), this is not available, so the values are simply expected to be higher, which does no harm.

TOPIC. The kinematic and dynamic methods, are they the different experimental methods or data treatment methods?  Which one is preferred and can they be done simultaneously and compare?

Explanation. Dynamic diffraction can also appear in X-ray diffraction but is typically small and can be ignored. In MicroED dynamic diffraction is the norm and can be treated during the data refinement process. 
Olex2 or Jana2020 can allow for dynamic refinement.
It would certainly be best to treat each and every dataset with dynamic refinement but is usually only conducted when determining absolute structures. It furthermore helps finding hydrogen atom positions, in case they cannot be found using kinematic treatment.

They are data treatment methods, in the final structure refinement step. Even in the dynamical method you will usually first do a kinematical refinement, so you will always have a comparison.

TOPIC. When grinding the sample, how can you make sure you haven't lost crystallinity?

Explanation. You don't normally lose crystallinity you are just crushing the large crystals into smaller crystalline grains but you could check with PXRD that you still have a pattern (if there is enough material) or just try it in the ED and see.

TOPIC. Does ED encounter twinning problems?

Explanation. It is absolutely possible; they are just very small single crystals after all. However, since you have thousands of grains to choose from, you would normally not need to handle it. Just ignore the twinned grains for an easier life.

TOPIC. If we have a crystal, good enough to perform electron density analysis up to 0.4 angstrom resolution.  By decreasing the size of same crystal through grinding and performing electron diffraction. Which method should be preferably used, single crystal X-ray diffraction or electron diffraction?

Explanation. I assume you are talking about doing charge density with X-rays in your first sentence? As a general rule X-ray diffraction is the best choice if you have crystals that are suitable. It makes no sense to crush them for electron diffraction. Electron diffraction is for samples which do not work with X-rays. X-rays first, electrons as a fall back.

This sounds to me like a case where you'd just do single-crystal X-ray diffraction, unless you are looking for something that you need specifically electrons for (e.g., measuring the EDT instead of electron density - which may give you more information about charge states or protonation)

TOPIC. What is the max tilt angle? Is "missing wedge" a problem for structure determination?

Explanation. The maximum tilt angle is typically limited by the geometry of the sample holder, e.g. +/- 80 deg.

The missing wedge can be a problem, e.g. in low symmetry space groups like P1 and P-1. Here data of multiple crystallites can be merged.

TOPIC. Does it matter which side of the TEM grid faces towards upcoming electron beam?

Explanation. No, it really does not matter data Topicuality wise.

TOPIC. Colleague here is using Jeol TEM/STEM 200kV and tried collecting data like for structure solution in (rare) free time. Very complicated process of collecting and trying to process the data through train of programs for anything usable- ideally ins and hkl files for solution and refinement. XtaLAB Synergy-ED with CrysAlisPro integrated package shown in the video makes it look like simple, accessible, straightforward and fast thing to do.

Explanation. Thanks, we are fortunate to have talented people working on these problems.

TOPIC. Can microED/3DED also be used to determine crystallite size?

Explanation. Absolutely! The electron diffractometer still has imaging capabilities.

TOPIC. Will the refined result file still call .cif and I wonder will ppl still use checkcif to validate it?  or what to do in order to tell ppl the result is a 'reasonable' one?

Explanation. At the end of analysis you will have a .cif file at hand that checkCIF can process. CheckCIF recognizes that data is from MicroED and conducts structure validation accordingly. With MicroED come a few new entries that can be used, please refer to CIF dictionary. 
New entries are becoming available from time to time and CrysAlisPro it set up for this.

TOPIC. Could you please comment on how air-sensitive compounds should be handled when preparing microcrystals for ED experiments? In particular, it is not possible to use oil to protect the sample while transferring it to the diffractometer, right?

Explanation. There are special holders for the sample. There are cryo transfer holders which let you load the sample in a protective environment outside the diffractometer and freeze it before transfer to the diffractometer, but there are also air-free holders which can load the sample at room temperature without exposing to air. Oil is a bad idea; it will absorb the electron beam.

TOPIC. Will the missing wedge angle problem limit the space group symmetry? Since low symmetry reTopicuires more data at different angles.

Explanation. You can merge (combine) datasets of multiple crystals to overcome completeness issues in low symmetry space groups. You see it demonstrated just now.

TOPIC. Just wonder where can I find the R(rim) in Olex2?

Explanation. In data finalization window, you can enable "Printout Options". It is located on the right side, roughly in the middle. I cannot share a screenshot here unfortunately. You can have a wide range of data Topicuality indicators and also define more than 10 resolution slices for the data.
Though ppl are still focus on R(int) mostly, even during the checkcif validation process.  Comment have to be made for merged data (twin data)

TOPIC. Is the data collection done for several crystals and merge the data, or for a good one crystal? In the video shows kind of screen for different crystals, is it just to screen and select a good one?  I could not get it.

Explanation. Different crystals. We are trying to stitch together a complete dataset and this reTopicuires crystals in different orientations so that we can access reflections which are not accessible in a given grain. This is a common technical Topicue in protein crystallography also but more because of beam damage destroying the crystals (which is also a valid reason in ED)

TOPIC. PLEASE THE DIFERENCE BETWEEN THIS TECHNITOPICUE AND SYNCHROTRON

Explanation. Synchrotron uses an X-ray beam just like a home lab instrument the only differences are that the beam is more intense, more parallel, is monochromatically purer, and can be tuned to a given wavelength. In this lecture we are talking about electron diffraction, this techniTopicue replaces the X-ray beam with an electron beam. Electrons also behave like waves and diffract from the crystal. Electrons interact more strongly with matter than X-rays so despite being lower power, strong diffraction can be observed.

TOPIC. To obtain reliable absolute structure- are there some stricter limits on R_int, Rmerge, R and resolution?

Explanation. What matters most is redundancy and Topicuality. You should ensure I/sigma is good and you have good redundancy. Higher resolution than you might normally collect might be helpful to improve data-to-parameter ratio. It is likely that a high R1/R_int etc. structure has an unreliable Flack, but as long as they are within normal ranges, you should be ok

High I/sig(I) helps with the Flack parameter and high redundancy in the dataset helps with the standard deviation of the Flack parameter. So you need both, high intensity data and high redundancy (or a heavy element with resonant scattering). If you look at light atom structures (HCNO) the crystal must be chosen very well, as intensity differences of the Friedel pairs are low.

TOPIC. What does e.s.d stand for in the previous slide?

Explanation. estimated standard deviation

TOPIC. less than 2 esd in X means?

It means that the error on the Flack parameter should not be higher than twice the Flack parameter itself. Ideally you achieve x = 0.01(2) or better

TOPIC. If a compound has 1000 carbon atoms but only one heavy rare-earth atom, in this case, which wavelength, Mo or Cu is suggested for absolute structure determination?

Explanation. A crystal structure as you describe should not exhibit great diffraction strength and Cu radiation is the best choice in this case, as absorption will be low and resonant scattering is high.

In this extreme case, is the only one heavy atom helpful for anomalous scattering under Mo radiation?

You should get a proper Flack parameter too.

TOPIC. In monoclinic system space group P21/n why after solving get Z=2 because for monoclinic system Z=4.

Explanation. What is in the asymmetric unit? Do you see partial molecules? Sometimes you have a molecule lying across a symmetry element and you only see half of it. Maybe you have half molecules in your Asymmetric unit and we normally report the formula for the chemical moiety.  So when you expand the formula to describe the molecule, Z must be altered.

Half molecule in the asymmetric unit.

TOPIC. If I have Hooft Parameter -0.01(6) and a Flack Parameter that is essentially meaningless (0.00(8) ), can I say that I can determine absolute stereochemistry?

Explanation. In that case you determined the absolute structure satisfyingly well. 0.00(8) is a perfect result!

Remember the esd of 8 is on the last decimal place.

To make it even more clear:
0.00(8) means 0.00 +/- 0.008

TOPIC. Is mean the value of F2+-F2-/mig higher the better?

Explanation. Yes, the higher the easier it is to obtain a good Flack parameter. 
The value can be improved by measuring a high intensity dataset, adding high redundancy or a heavy atom like P, S, Cl, metal, etc.

TOPIC. are the Flack parameters of -0.04(8) or 0.03(8) acceptable to establish reliable absolute structures?

Explanation. For ab initio absolute structure determination Flack parameter should be smaller than x = 0.04(8).
If some chiral center is known, e.g. from synthesis, you can live with a higher value too.

During CIF file preparation one would enter additional information:
E.g. Absolute structure was determined by anomalous dispersion, reference molecule, reference molecule + anomalous dispersion, synthesis, unknown, not applicable

TOPIC. In the Ylid example now, the flack (-0.01(5)) for the second dataset is not good enough because the standard deviation (5) is > 2??

Explanation.  I mean x > 2 as mentioned previously in the slides

This is definitely good enough -- besides, 'Trust the Hooft Parameter'!

Maybe a misunderstanding. x is the flack parameter, in your example -0.01, the esd is 0.05. The flack parameter should less than half of the esd value. detailed analysis by flack himself.

TOPIC. This is a version of Olex2 specifically designed for ED? or it is normal one available from the olex2 website?

Explanation. It's the normal version of Olex2, but the datasets need to be processed with a licenced version of CAP -- contact Rigaku for more details.

TOPIC. How to reduce the hooft factor in refinement?

Explanation. Basically, there's nothing you can do after the data collection. You may need to collect new data if you don't get a conclusive result

So while strategy we have to increase the redundancy